EMBO

Embolization Particle Sizing System

Custom MCU Board — PCB Design Brief

| Field | Detail |
| --- | --- |
| Document version | v2.4 — UI redesign: TFT + EC11 encoder + 2 buttons + buzzer on separate breakout board connected via 20-pin IDC ribbon cable |
| Date | June 2026 |
| Status | For schematic design |
| MCU module | ESP32-S3-WROOM-1-N8 (8MB flash, PCB antenna, no PSRAM) |
| Programming | USB-C direct GPIO19/20 — no USB-UART bridge chip needed |
| Wireless debug | Bluetooth LE 5.0 on-chip — NimBLE BLE UART |
| Motor drivers | 2× TMC2209-LA QFN-28 — shared UART GPIO4, StallGuard viscosity sensing |
| Motor connectors | 2× JST-XH 6-pin — pin order A1/NC/A2/B1/NC/B2 |
| RPi UART | JST-XH 2.5mm 3-pin — GND(1)/RX(2)/TX(3) |
| UI system | Breakout board via 20-pin IDC ribbon: ILI9341 TFT, EC11 encoder, 2 buttons, passive piezo buzzer |
| Reverse polarity | AO4407A SOIC-8 P-MOSFET + BZX84C15 15V Zener gate clamp |
| Fabrication | JLCPCB — 2-layer FR4 ENIG, hot plate reflow with paste and stencil |

v2.4 changes from v2.3: UI completely redesigned — TFT module, EC11 rotary encoder, 2 push buttons, and passive piezo buzzer moved to a separate UI breakout board connected to the main board via 20-pin 2×10 IDC ribbon cable. Main board J9 changes from 14-pin TFT socket to 20-pin IDC box header. J10/J11/J12 (three 3-pin button headers) removed. GPIO11→BTN1, GPIO12→BTN2, GPIO13→BUZ_PWM added. GPIO16/17/18 repurposed from buttons to EC11_A/EC11_B/EC11_SW. SPI clock note added: reduce from 40MHz to 20MHz in firmware for signal integrity through ribbon cable.

# 1. System overview

EMBO is a closed-loop embolic agent preparation device. The MCU board integrates three sensor subsystems (UAS, CV camera, StallGuard viscosity), two stepper motor actuators, two mechanical limit switches, and a UI breakout board connected via IDC ribbon cable.

## 1.1 Why ESP32-S3-WROOM-1-N8

- Module: chip, 8MB flash, 40MHz crystal, PCB antenna — no external MCU support components needed

- Built-in USB Serial/JTAG on GPIO19/20 — no bridge IC needed

- Bluetooth LE 5.0 on-chip — NimBLE for wireless debug

- 36 GPIO broken out — 28 used in v2.4, 8 free

- N8 variant: no PSRAM — GPIO35–37 free for SPI bus

## 1.2 Power rails

| Rail | Source | Voltage | Consumers | Max current |
| --- | --- | --- | --- | --- |
| 12/24V | PSU direct | 12–24V | 2× TMC2209 VM | 6A peak |
| 5V | LM2596 buck | 5V ±2% | LED panel, TMC2209 VCC_IO, buzzer (via ribbon) | 2A |
| 3.3V | AMS1117-3.3 LDO | 3.3V ±1% | ESP32, AD9833, IC1/IC2, TFT logic (via ribbon) | 800mA |

# 2. ESP32-S3-WROOM-1-N8 module

## 2.1 Footprint and keepout

- Dimensions: 18.0×25.5×3.1mm. Castellated pads, 1.27mm pitch.

- ANTENNA KEEPOUT: no copper, traces, vias, or components within 15mm of PCB antenna on ALL layers.

- EPAD: solder to GND pour.

## 2.2 Power decoupling

| Pin | Net | Decoupling |
| --- | --- | --- |
| VDD3P3 (×2) | 3.3V | 10µF X5R 0805 + 100nF X7R 0402 per pin — within 2mm |
| VDD3P3_RTC | 3.3V | 100nF X7R 0402 within 2mm |
| GND (multiple) | GND | All to ground pour |

## 2.3 EN reset + boot circuits

| Component | Value | Connection |
| --- | --- | --- |
| R_en | 10kΩ 0402 | 3.3V → EN pin |
| C_en | 1µF X7R 0402 | EN → GND |
| SW_reset | 6×6mm tactile | EN → GND. Mount on board edge. |
| R_boot | 10kΩ 0402 | 3.3V → GPIO0 |
| C_boot | 100nF X7R 0402 | GPIO0 → GND |
| SW_boot | 6×6mm tactile | GPIO0 → GND. Hold during reset for bootloader. |

## 2.4 USB-C programming connector (J13)

| USB-C signal | PCB connection | Notes |
| --- | --- | --- |
| D− A-side (A7) | GPIO19 | Tie A7+B7 together on D− net → GPIO19. No series resistor. |
| D− B-side (B7) | GPIO19 (same net) | A7 and B7 shorted on PCB for cable-flip compatibility. |
| D+ A-side (A6) | GPIO20 | Tie A6+B6 together on D+ net → GPIO20. No series resistor. |
| D+ B-side (B6) | GPIO20 (same net) | A6 and B6 shorted on PCB. |
| CC1 (A5) | 5.1kΩ to GND | Mandatory for USB-C charger detection. |
| CC2 (B5) | 5.1kΩ to GND | Mandatory for USB-C charger detection. |
| VBUS | 5V rail via 1N5819 | USB-powered operation during development. |
| GND | GND |  |
| SBU1 (A8) | NC — no connect | Not required for USB 2.0. Leave open. |
| SBU2 (B8) | NC — no connect | Not required for USB 2.0. Leave open. |
| SSTX / SSRX | NC — no connect | SuperSpeed pairs unused. Leave open. |

GPIO19 and GPIO20 are fixed USB PHY pins — never connect any other signal. In schematic: D+ net connects to both A6 and B6 pads. D− net connects to both A7 and B7 pads.

# 3. Power section

## 3.1 Reverse polarity protection — AO4407A + BZX84C15 Zener

| Component | Value / Part | Connection |
| --- | --- | --- |
| Q1 | AO4407A SOIC-8 P-ch MOSFET −30V −12A | Source (pins 2–4) → J14 V+. Drain (pins 5–8) → protected 24V rail. Gate (pin 1) → R_gate/D7 junction. |
| R_gate | 10kΩ 0402 | Gate → GND. |
| D7 | BZX84C15 15V Zener SOT-23 | Cathode → Source (24V+). Anode → Gate. Clamps Vgs = −15V. Without D7: Vgs = −24V exceeds AO4407A ±20V Vgs_max. |

## 3.2 Input protection (after Q1)

| Component | Value | Connection |
| --- | --- | --- |
| F1 | 1A polyfuse (1812) | In series on 24V rail after Q1 |
| D5 | SMBJ28A TVS | Across 24V rail after F1 |
| Power LED | Red LED + 10kΩ | 3.3V → 10kΩ → LED → GND |

## 3.3 Buck: 24V → 5V (LM2596-5.0)

| Parameter | Requirement |
| --- | --- |
| IC | LM2596-5.0 fixed output |
| Input | 8–40V |
| Output | 5V 3A |
| Inductor | 100µH ≥3A saturation low DCR |
| Input cap | 100µF ≥40V electrolytic + 100nF ceramic |
| Output cap | 220µF low-ESR electrolytic + 100nF ceramic |
| Catch diode | SS34A DO-214AB SMA — within 5mm of SW pin, minimum loop area |
| SW snubber | 10Ω + 1nF from SW to GND |
| Bleed resistor | 4.7kΩ from 5V to GND |

## 3.4 LDO: 5V → 3.3V (AMS1117-3.3)

| Parameter | Requirement |
| --- | --- |
| IC | AMS1117-3.3 SOT-223 |
| Output | 3.3V 800mA max |
| Input cap | 10µF tantalum Case A |
| Output cap | 10µF tantalum Case A + 100nF ceramic |
| Thermal — CRITICAL | SOT-223 tab to ≥100mm² copper pour + ≥4 thermal vias to bottom GND plane. ≈850mW at full load. |

# 4. GPIO pin assignments

SPI mode: AD9833 uses Mode 2 (CPOL=1, CPHA=0). ILI9341 and XPT2046 use Mode 0 (CPOL=0, CPHA=0). Firmware must switch mode before each AD9833 transaction. XPT2046 max SPI clock: 2MHz. ILI9341 over ribbon cable: reduce from 40MHz to 20MHz to maintain signal integrity through IDC ribbon capacitance.

| GPIO | Signal | Dir | Connected to | Notes |
| --- | --- | --- | --- | --- |
| 1 | UAS_ADC | Analog in | Envelope detector output | ADC1_CH0. Safe with BLE. 100nF C_adc to AGND within 2mm. |
| 2 | STATUS_LED | Out | 330Ω → green LED → GND | GPIO out |
| 4 | TMC_UART | Bidir | TMC2209 PDN_UART junction (1kΩ + 10kΩ pull-up) | UART1 TX+RX half-duplex. See Section 6.2. |
| 5 | STEP_M1 | Out | TMC2209 U5 STEP | LEDC PWM |
| 6 | DIR_M1 | Out | TMC2209 U5 DIR | GPIO out |
| 7 | EN_M1 | Out | TMC2209 U5 EN | 10kΩ pull-up at IC |
| 8 | STEP_M2 | Out | TMC2209 U6 STEP | LEDC PWM |
| 9 | DIR_M2 | Out | TMC2209 U6 DIR | GPIO out |
| 10 | EN_M2 | Out | TMC2209 U6 EN | 10kΩ pull-up at IC |
| 11 | BTN1 | In | IDC ribbon pin 16 → breakout button 1 | Internal pull-up. Pull-up + 100nF debounce also on breakout. |
| 12 | BTN2 | In | IDC ribbon pin 17 → breakout button 2 | Internal pull-up. Pull-up + 100nF debounce also on breakout. |
| 13 | BUZ_PWM | Out | IDC ribbon pin 18 → breakout buzzer driver gate | LEDC PWM for tone generation. 2N7002 MOSFET gate on breakout. |
| 14 | LIMIT_M1 | In | Limit switch 1 → GND (J6) | Internal pull-up. 100nF debounce. |
| 15 | LIMIT_M2 | In | Limit switch 2 → GND (J7) | Internal pull-up. 100nF debounce. |
| 16 | EC11_A (CLK) | In | IDC ribbon pin 13 → EC11 pin A | Internal pull-up. Pull-up + 100nF debounce on breakout. |
| 17 | EC11_B (DT) | In | IDC ribbon pin 14 → EC11 pin B | Internal pull-up. Pull-up + 100nF debounce on breakout. |
| 18 | EC11_SW | In | IDC ribbon pin 15 → EC11 push switch | Internal pull-up. Pull-up + 100nF debounce on breakout. |
| 19 | USB D− | Fixed | USB-C J13 A7+B7 | Fixed USB PHY. No other connection. |
| 20 | USB D+ | Fixed | USB-C J13 A6+B6 | Fixed USB PHY. No other connection. |
| 21 | TOUCH_IRQ | In | IDC ribbon pin 12 → XPT2046 T_IRQ | Internal pull-up. 100nF to GND on main board near IDC header. |
| 35 | SPI_MOSI | Out | AD9833 SDATA + IDC ribbon pin 5 (TFT + touch) | SPI2. Mode 0 display/touch, Mode 2 AD9833. |
| 36 | SPI_CLK | Out | AD9833 SCLK + IDC ribbon pin 7 (TFT + touch) | SPI2. 20MHz display via ribbon, 2MHz touch, ~10MHz AD9833 direct. |
| 37 | SPI_MISO | In | IDC ribbon pin 6 (touch MISO) | SPI2. ILI9341 write-only, XPT2046 readback via ribbon. |
| 38 | AD9833_CS | Out | AD9833 FSYNC (active low) | GPIO out. Assert before SPI Mode 2 transaction. |
| 39 | TFT_CS | Out | IDC ribbon pin 8 → ILI9341 CS | GPIO out |
| 40 | TFT_DC | Out | IDC ribbon pin 9 → ILI9341 DC | GPIO out |
| 41 | TFT_RST | Out | IDC ribbon pin 10 → ILI9341 RESET | GPIO out |
| 42 | TOUCH_CS | Out | IDC ribbon pin 11 → XPT2046 T_CS | GPIO out |
| 47 | RPi_TX | Out | J3 pin 3 → RPi GPIO15 | UART1 TX 3.3V 921600 baud |
| 48 | RPi_RX | In | J3 pin 2 ← RPi GPIO14 | UART1 RX 3.3V |

# 5. UAS signal chain

1MHz acoustic signal chain for particle attenuation. All on 3.3V rail. AD9833 can sweep 0.5–3MHz in firmware — same hardware.

## 5.1 AD9833BRMZ — signal generator

| Parameter | Requirement |
| --- | --- |
| IC | AD9833BRMZ (MSOP-10) |
| Supply | 3.3V — both VDD pins |
| SPI | GPIO38 (FSYNC), GPIO36 (SCLK), GPIO35 (SDATA). Write-only. SPI Mode 2. |
| RSET | 6.8kΩ 1% 0402 to AGND → 604mVpp across 200Ω load |
| COMP | 10nF X7R 0402 to AGND |
| Output coupling | 100nF X7R from IOUT load to Tx amp input |
| Decoupling | 100nF X7R per VDD pin + 10µF X5R 0805 bulk |

## 5.2 Tx amplifier — IC1 ch-A, G = 4.9

| Parameter | Requirement |
| --- | --- |
| IC | OPA2354 SOIC-8 ch-A of IC1. 250MHz GBW. 3.3V supply. |
| Bias | Two 10kΩ (3.3V → non-inv → GND) — 1.65V mid-rail |
| Gain | Rf = 390Ω 1%, Rg = 100Ω 1%. G = 4.9. Output = 2.96Vpp peak = 3.13V < 3.3V ✓ |
| Output | 47Ω series + 100nF coupling to J1 SMA |
| Decoupling | 100nF within 2mm V+ + 10µF bulk |

## 5.3 IC1 ch-B — unused, tied off

- Pin 5 (non-inv): connect to 1.65V mid-rail node

- Pin 6 (inv): connect to pin 7 (output) — unity-gain follower. Prevents oscillation.

## 5.4 Rx input protection — J2 SMA → R_in → D1 BAV99

| Component | Value | Function |
| --- | --- | --- |
| R_in | 100Ω 0402 | Series fault current limiter |
| D1 | BAV99 SOT-23 | Dual diode clamp 0V–3.3V at IC2 input |

## 5.5 Rx two-stage amplification — IC2, G = 100 total

| Stage | Pin | Rf | Rg | Gain | Notes |
| --- | --- | --- | --- | --- | --- |
| Rx stage 1 | IC2 ch-A | 910Ω 1% | 100Ω 1% | 10.1× | 100nF in, 100nF inter-stage, bias divider 10kΩ/10kΩ |
| Rx stage 2 | IC2 ch-B | 910Ω 1% | 100Ω 1% | 10.1× | Total 100× (+40dB). Max non-clip Rx input: 32mVpp. |

## 5.6 Envelope detector and ADC input — GPIO1

| Component | Value | Function |
| --- | --- | --- |
| D2 | BAT54 SOD-123 | Envelope rectifier Vf≈0.2V. Anode: IC2 ch-B output. Cathode: R_env/C_env junction. |
| R_env | 10kΩ 0402 | RC τ=100µs, fc=1.6kHz, ripple at 1MHz=0.16%, settling 5τ=500µs. |
| C_env | 10nF X7R 0402 | RC filter cap parallel with R_env. |
| D3 | BAV99 SOT-23 | Hard clamp at GPIO1 — overvoltage protection. |
| C_adc | 100nF X7R 0402 | GPIO1 to AGND. Per Espressif ADC input recommendation. Within 2mm of ESP32 GPIO1 pad. Separate from C_env. |

No voltage divider at GPIO1. Max envelope ≈1.4V. ESP32 ADC ADC_ATTEN_DB_11 mode: 0–3.1V range, 0.757mV/LSB. Firmware: wait ≥500µs after each AD9833 frequency step before ADC read.

# 6. Motor drivers — 2× TMC2209-LA

Two TMC2209-LA (QFN-28) drive two NEMA17 motors. Hot plate reflow required for QFN-28. Shared half-duplex UART on GPIO4. StallGuard4 viscosity proxy via SG_RESULT in DRVSTATUS.

## 6.1 UART addressing

| Driver | Ref | MS1 | MS2 | Address |
| --- | --- | --- | --- | --- |
| Motor 1 | U5 | GND | GND | 0 |
| Motor 2 | U6 | 3.3V via 10kΩ | GND | 1 |

## 6.2 UART junction and PDN pull-up

| Component | Value | Connection |
| --- | --- | --- |
| R_UART | 1kΩ 0402 | GPIO4 → PDN_UART junction node. |
| R_PDN_UP | 10kΩ 0402 | PDN_UART junction → 3.3V. Prevents TMC2209 power-down during ESP32 boot transients. |
| Junction | — | Both U5 and U6 PDN_UART connect here. Short equal-length traces from junction to each IC. |

## 6.3 Power and critical pins — each TMC2209

| Pin | Connection | Notes |
| --- | --- | --- |
| VM | 24V + 100µF + 100nF per IC | Separate caps per IC — not shared. Absorbs motor back-EMF. |
| VCC_IO | 3.3V | Logic supply |
| VCP | 100nF X7R to GND — CRITICAL | Charge pump cap. Without this the motor will not run. Within 2mm. |
| SPREAD | 3.3V via 10kΩ | Hardware SpreadCycle. StallGuard requires SpreadCycle mode. |
| EN (ENN) | 10kΩ pull-up to 3.3V at IC | Active LOW. Pull-up overrides boot glitch. |
| CLK | Floating | Internal 12MHz oscillator. |
| VREF | Floating | UART current control. |
| Thermal pad | GND pour via ≥4 vias | Test GND continuity after reflow. |

## 6.4 Control — U5 (Motor 1) and U6 (Motor 2)

| Signal | U5 GPIO | U6 GPIO | Notes |
| --- | --- | --- | --- |
| STEP | GPIO5 | GPIO8 | LEDC PWM |
| DIR | GPIO6 | GPIO9 | GPIO out |
| EN | GPIO7 | GPIO10 | 10kΩ pull-up at IC |
| PDN_UART | GPIO4 (shared) | GPIO4 (shared) | Via R_UART + R_PDN_UP junction |

## 6.5 Motor output connectors — JST-XH 6-pin (J4, J5)

| Ref | Label | Pin 1 | Pin 2 | Pin 3 | Pin 4 | Pin 5 | Pin 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| J4 | Motor 1 | OA1→U5 | NC | OA2→U5 | OB1→U5 | NC | OB2→U5 |
| J5 | Motor 2 | OA1→U6 | NC | OA2→U6 | OB1→U6 | NC | OB2→U6 |

RS1/RS2 = 0.11Ω 1% 2512 on U5 sense pins. RS3/RS4 = 0.11Ω 1% on U6. Kelvin connections. Motor traces ≥0.5mm. NC pins (2, 5): connect to GND or confirm NC — no floating copper.

# 7. Limit switches

| GPIO | Ref | Connector | Wiring |
| --- | --- | --- | --- |
| GPIO14 | LIMIT_M1 | J6 JST-XH 2.5mm 2-pin | Pin 1: GND. Pin 2: GPIO14. 100nF debounce. |
| GPIO15 | LIMIT_M2 | J7 JST-XH 2.5mm 2-pin | Pin 1: GND. Pin 2: GPIO15. 100nF debounce. |

# 8. UI interface system — breakout board via IDC ribbon

All UI components (ILI9341 TFT, EC11 rotary encoder, 2 push buttons, passive piezo buzzer) are on a separate breakout PCB. The main board connects to the breakout via a 20-conductor 2.54mm IDC ribbon cable. This keeps high-frequency SPI switching noise away from the analog UAS section on the main board.

## 8.1 IDC ribbon cable — 20-pin 2×10, 2.54mm pitch

Standard IDC ribbon cable, 20 conductors. Both PCBs have IDC 2×10 shrouded box headers with polarisation key. Pin 1 is identified by the red stripe on the ribbon. Always route pin 1 to the same physical position on both connectors when crimping.

| Pin | Signal | Main board GPIO / net | Direction | Notes |
| --- | --- | --- | --- | --- |
| 1 | GND | GND | — | Pin 1 = red stripe reference. Always GND on pin 1. |
| 2 | 3.3V | 3.3V rail | Power out | Powers TFT logic and breakout pull-ups via ribbon. |
| 3 | GND | GND | — | Additional GND near power pins. |
| 4 | 5V | 5V rail | Power out | Buzzer drive supply. Louder at 5V than 3.3V. |
| 5 | SPI_MOSI | GPIO35 | Out | Shared: ILI9341 SDI + XPT2046 T_DIN. |
| 6 | SPI_MISO | GPIO37 | In | Shared: ILI9341 SDO + XPT2046 T_DO. |
| 7 | SPI_CLK | GPIO36 | Out | Shared: ILI9341 SCK + XPT2046 T_CLK. Firmware: 20MHz max for display via ribbon. |
| 8 | TFT_CS | GPIO39 | Out | ILI9341 chip select — active LOW. |
| 9 | TFT_DC | GPIO40 | Out | ILI9341 data/command select. |
| 10 | TFT_RST | GPIO41 | Out | ILI9341 reset — active LOW. |
| 11 | TOUCH_CS | GPIO42 | Out | XPT2046 chip select — active LOW. |
| 12 | TOUCH_IRQ | GPIO21 | In | XPT2046 interrupt — active LOW. 100nF to GND on main board near IDC header. |
| 13 | EC11_A (CLK) | GPIO16 | In | Encoder channel A. Pull-up + 100nF debounce on breakout. |
| 14 | EC11_B (DT) | GPIO17 | In | Encoder channel B. Pull-up + 100nF debounce on breakout. |
| 15 | EC11_SW | GPIO18 | In | Encoder push switch. Pull-up + 100nF debounce on breakout. |
| 16 | BTN1 | GPIO11 | In | Push button 1. Pull-up + 100nF debounce on breakout. |
| 17 | BTN2 | GPIO12 | In | Push button 2. Pull-up + 100nF debounce on breakout. |
| 18 | BUZ_PWM | GPIO13 | Out | Buzzer PWM drive → 2N7002 MOSFET gate on breakout. LEDC PWM. |
| 19 | GND | GND | — | Additional GND near signal return paths. |
| 20 | NC | — | — | Spare — no connect on both boards. |

Ribbon cable polarisation: IDC box headers have a plastic key that prevents reverse insertion. Always use shrouded (box) headers, not plain pin headers, for this connector on both boards. Mark pin 1 with a triangle or dot on the silkscreen of both PCBs.

## 8.2 Main board IDC connector (J9)

- Footprint: IDC 2×10 shrouded box header, 2.54mm pitch, vertical mount on main board

- All 20 ribbon signals route from their respective GPIO pads and power rails to the IDC header pads

- Place J9 on the board edge nearest the enclosure panel cutout for the ribbon cable to exit cleanly

- 100nF cap from pin 12 (TOUCH_IRQ) trace to GND on main board — filters ribbon cable noise at GPIO21

## 8.3 SPI signal integrity through ribbon cable

| Device | SPI mode | Clock (v2.3) | Clock (v2.4 via ribbon) | Reason |
| --- | --- | --- | --- | --- |
| ILI9341 display | Mode 0 | 40MHz | 20MHz max | Ribbon ≈50–100pF/m capacitance causes signal rounding at 40MHz. 20MHz maintains clean edges. |
| XPT2046 touch | Mode 0 | 2MHz | 2MHz (unchanged) | Already at maximum rated frequency. No change. |
| AD9833 DDS | Mode 2 | ~10MHz | ~10MHz (unchanged) | AD9833 is on main board — short direct trace, not through ribbon. |

## 8.4 Breakout board — component list

The UI breakout board is a separate PCB that hosts the following components. It receives power and signals from the main board via the 20-pin IDC ribbon and has no direct PSU connection of its own.

| Component | Qty | Notes |
| --- | --- | --- |
| IDC 2×10 box header 2.54mm | 1 | Female or male depending on cable assembly. Pin 1 marked on silkscreen. |
| ILI9341 TFT module socket (14-pin 2.54mm female) | 1 | Same pinout as v2.3 Section 8.4 14-pin table. See Section 8.5 below. |
| EC11 rotary encoder (5-pin, through-hole) | 1 | Two quadrature pins (A, B) + push switch (SW) + two GND/COM pins. |
| Push button (6×6mm tactile, through-hole) | 2 | BTN1 and BTN2. |
| Passive piezo buzzer | 1 | Through-hole radial. Driven by 2N7002 MOSFET. Do not use active (self-oscillating) buzzer — PWM tone control requires passive type. |
| 2N7002 N-ch MOSFET SOT-23 | 1 | Buzzer driver. Gate → 10kΩ from BUZ_PWM (ribbon pin 18). Source → GND. Drain → buzzer − . Buzzer + → ribbon 5V (pin 4). |
| 10kΩ 0402 resistors | 6 | Pull-ups: EC11_A, EC11_B, EC11_SW, BTN1, BTN2 (×5). Buzzer gate resistor (×1). |
| 100nF X7R 0402 caps | 5 | Debounce: EC11_A, EC11_B, EC11_SW, BTN1, BTN2 — each cap from signal to GND. |
| 47Ω 0402 resistor | 1 | TFT backlight LED series resistor. From 3.3V (ribbon pin 2) to TFT module LED pin. |
| 10kΩ 0402 resistor | 1 | TFT RST pull-up. From 3.3V (ribbon pin 2) to TFT RESET pin (socket pin 4). |

## 8.5 TFT module connection on breakout (14-pin socket)

| Socket pin | Module label | Signal from ribbon / net | Notes |
| --- | --- | --- | --- |
| 1 | VCC | Ribbon pin 2 (3.3V) | Module powered from 3.3V via ribbon. |
| 2 | GND | Ribbon pin 1 or 3 or 19 (GND) |  |
| 3 | CS | Ribbon pin 8 (TFT_CS/GPIO39) | Active LOW |
| 4 | RESET | Ribbon pin 10 (TFT_RST/GPIO41) | 10kΩ pull-up to 3.3V on breakout. |
| 5 | DC | Ribbon pin 9 (TFT_DC/GPIO40) | Data/command select |
| 6 | SDI (MOSI) | Ribbon pin 5 (SPI_MOSI/GPIO35) | Shared SPI bus |
| 7 | SCK | Ribbon pin 7 (SPI_CLK/GPIO36) | 20MHz max via ribbon |
| 8 | LED | 3.3V via 47Ω on breakout | Always-on backlight. 47Ω on breakout board. |
| 9 | SDO (MISO) | Ribbon pin 6 (SPI_MISO/GPIO37) | XPT2046 readback. ILI9341 write-only. |
| 10 | T_CLK | Ribbon pin 7 (same net as SCK) | XPT2046 clock. Tie to pin 7 net on breakout. |
| 11 | T_CS | Ribbon pin 11 (TOUCH_CS/GPIO42) | XPT2046 chip select |
| 12 | T_DIN | Ribbon pin 5 (same net as MOSI) | Tie to pin 6 net on breakout. |
| 13 | T_DO | Ribbon pin 6 (same net as MISO) | Tie to pin 9 net on breakout. |
| 14 | T_IRQ | Ribbon pin 12 (TOUCH_IRQ/GPIO21) | Active LOW interrupt. 100nF to GND on main board side. |

On the breakout PCB schematic: create net MOSI → connect socket pins 6 and 12. Create net CLK → connect socket pins 7 and 10. Create net MISO → connect socket pins 9 and 13. These are correctly shared between ILI9341 and XPT2046.

## 8.6 EC11 rotary encoder

- 5-pin through-hole component. Pins: A (CLK), B (DT), SW (push switch), COM (GND) × 2

- A (CLK) → ribbon pin 13 (GPIO16). Pull-up 10kΩ to 3.3V + 100nF to GND on breakout.

- B (DT) → ribbon pin 14 (GPIO17). Pull-up 10kΩ to 3.3V + 100nF to GND on breakout.

- SW (push) → ribbon pin 15 (GPIO18). Pull-up 10kΩ to 3.3V + 100nF to GND on breakout.

- Both COM pins → GND on breakout.

Firmware: use hardware interrupt (EXTI) on both A and B for quadrature decoding. Software debounce (5ms) required for SW push. Encoder bounce on A/B is handled by the quadrature state machine — do not over-filter A/B or rotation steps will be missed at speed.

## 8.7 Push buttons — BTN1 and BTN2

- Each: 6×6mm tactile switch, one pin to GND, one to signal and pull-up.

- BTN1 signal → ribbon pin 16 (GPIO11). 10kΩ pull-up to 3.3V + 100nF to GND on breakout.

- BTN2 signal → ribbon pin 17 (GPIO12). 10kΩ pull-up to 3.3V + 100nF to GND on breakout.

- GPIO11 and GPIO12 internal pull-ups also enabled in firmware as backup.

## 8.8 Passive piezo buzzer and driver

| Component | Value | Connection |
| --- | --- | --- |
| Passive piezo buzzer | 5V rated, through-hole radial | Buzzer + terminal → ribbon 5V (pin 4). Buzzer − terminal → 2N7002 drain. |
| 2N7002 N-ch MOSFET | SOT-23 | Gate → 10kΩ resistor → ribbon BUZ_PWM (pin 18 / GPIO13). Source → GND. Drain → buzzer − terminal. |
| R_BUZ_GATE | 10kΩ 0402 | BUZ_PWM ribbon signal to 2N7002 gate. |

Use passive (not active) piezo buzzer — firmware uses LEDC PWM on GPIO13 to generate tones at specific frequencies. Active (self-oscillating) buzzers only produce one fixed frequency and cannot be frequency-controlled. Buzzer powered from 5V via ribbon pin 4 for adequate volume. 2N7002 Vgs(th) ≈ 1–2.5V — fully enhanced at 3.3V GPIO output.

## 8.9 Touch calibration

- XPT2046 returns raw 12-bit ADC values — first-run calibration maps to pixel coordinates

- Store mapping coefficients in ESP32 NVS (Non-Volatile Storage) — persists across power cycles

# 9. External connectors — complete list (v2.4)

| Ref | Label | Type | Pin assignment / connected to |
| --- | --- | --- | --- |
| J1 | TX Transducer | SMA PCB edge-launch | Centre: IC1 ch-A output after 47Ω + 100nF. Shell: AGND. |
| J2 | RX Transducer | SMA PCB edge-launch | Centre: R_in (100Ω) → D1 BAV99. Shell: AGND. |
| J3 | RPi UART | JST-XH 2.5mm 3-pin | Pin 1: GND. Pin 2: GPIO48 RX. Pin 3: GPIO47 TX. 921600 baud. No power to RPi. |
| J4 | Motor 1 | JST-XH 2.5mm 6-pin | Pin 1:OA1 Pin 2:NC Pin 3:OA2 Pin 4:OB1 Pin 5:NC Pin 6:OB2 — all to TMC2209 U5. |
| J5 | Motor 2 | JST-XH 2.5mm 6-pin | Pin 1:OA1 Pin 2:NC Pin 3:OA2 Pin 4:OB1 Pin 5:NC Pin 6:OB2 — all to TMC2209 U6. |
| J6 | Limit switch M1 | JST-XH 2.5mm 2-pin | Pin 1: GND. Pin 2: GPIO14. |
| J7 | Limit switch M2 | JST-XH 2.5mm 2-pin | Pin 1: GND. Pin 2: GPIO15. |
| J8 | LED panel | JST-XH 2.5mm 2-pin | Pin 1: 5V. Pin 2: GND. Always-on. |
| J9 | UI breakout IDC (CHANGED) | IDC 2×10 box header 2.54mm vertical | 20-pin ribbon to UI breakout board. Pin 1=GND (red stripe). See Section 8.1 for full assignment. |
| J13 | USB-C programming | USB-C receptacle | A6+B6→GPIO20, A7+B7→GPIO19, CC1+CC2 via 5.1kΩ, VBUS via D6. SBU1/SBU2 NC. |
| J14 | 24V PSU input | 3.5mm 2-pin screw terminal | Via Q1 AO4407A → F1 polyfuse → D5 TVS. |

J10/J11/J12 (three 3-pin button headers from v2.3) are removed. Buttons and encoder are now on the UI breakout board connected via J9 IDC ribbon.

# 10. PCB layout rules

## 10.1 Layer stack

- 2-layer FR4 1oz copper ENIG. Top: all SMD + signal routing. Bottom: ground plane + power routing.

- UAS analog signals on top layer only — no vias in UAS paths.

## 10.2 Antenna keepout

- No copper within 15mm of ESP32 antenna on ALL layers. Remove bottom copper in keepout zone.

- Place module at board edge with antenna outward.

## 10.3 Power section

- Q1 AO4407A: D7 Zener cathode to Source, anode to Gate, within 3mm of Q1.

- SS34A catch diode within 5mm of LM2596 SW pin. Snubber 10Ω+1nF within 2mm of SW pin.

- AMS1117: SOT-223 tab to ≥100mm² copper pour + ≥4 thermal vias.

## 10.4 UAS chain

- IC1 (Tx) and IC2 (Rx): ≥15mm separation. Keep motor traces away.

- J2 → R_in → D1 → IC2: under 20mm, no vias, GND guard each side.

- C_adc within 2mm of GPIO1 pad. C_env within 5mm of IC2 ch-B output.

## 10.5 TMC2209

- QFN-28: hot plate reflow with paste + stencil. VCP 100nF within 2mm per IC. Thermal pad GND continuity verified after reflow.

- VM decoupling (100µF + 100nF) directly at VM pin per IC — not shared.

- R_PDN_UP (10kΩ): place near UART junction node.

## 10.6 IDC ribbon connector (J9)

- Place J9 at board edge — ribbon needs to exit the enclosure to reach breakout board.

- 100nF cap from TOUCH_IRQ trace (IDC pin 12) to GND on main board — within 5mm of J9.

- Route SPI signals (IDC pins 5/6/7) away from TMC2209 motor outputs and UAS analog area.

- 3.3V and 5V ribbon traces: ≥0.3mm width to handle breakout board current load.

## 10.7 USB-C routing

- D+ (GPIO20): connect to both A6 and B6 pads of USB-C connector as single net.

- D− (GPIO19): connect to both A7 and B7 pads of USB-C connector as single net.

- CC1 and CC2 resistors within 5mm of connector.

# 11. Bill of materials — main board (v2.4)

| Qty | Ref | Description | Part | Package |
| --- | --- | --- | --- | --- |
| 1 | U1 | ESP32-S3 MCU module | ESP32-S3-WROOM-1-N8 | Castellated 18×25.5mm |
| 1 | Q1 | P-MOSFET reverse polarity | AO4407A | SOIC-8 |
| 1 | D7 | Gate Zener clamp | BZX84C15 15V | SOT-23 |
| 1 | U2 | Buck 24V→5V | LM2596-5.0 | TO-263-5 |
| 1 | U3 | LDO 5V→3.3V | AMS1117-3.3 | SOT-223 |
| 1 | U4 | DDS signal gen | AD9833BRMZ | MSOP-10 |
| 2 | IC1, IC2 | Dual op-amp UAS | OPA2354 | SOIC-8 |
| 2 | U5, U6 | Stepper driver | TMC2209-LA | QFN-28 reflow |
| 2 | D1, D3 | Dual diode clamp | BAV99 | SOT-23 |
| 1 | D2 | Envelope rectifier | BAT54 | SOD-123 |
| 1 | D4 | Buck catch diode | SS34A | DO-214AB SMA |
| 1 | D5 | TVS | SMBJ28A | DO-214AA SMA |
| 1 | D6 | USB Schottky | 1N5819 | SOD-123 |
| 1 | F1 | Polyfuse 1A | — | 1812 SMD |
| 1 | L1 | Buck inductor | 100µH ≥3A e.g. Bourns SRR1260 | SMD |
| 4 | RS1–RS4 | Current sense | 0.11Ω 1% | 2512 SMD |
| 2 | J1, J2 | SMA transducer | SMA PCB edge-launch | Board edge |
| 1 | J3 | RPi UART | JST-XH 2.5mm 3-pin | Vertical |
| 2 | J4, J5 | Motor connectors | JST-XH 2.5mm 6-pin | Vertical |
| 2 | J6, J7 | Limit switches | JST-XH 2.5mm 2-pin | Vertical |
| 1 | J8 | LED panel | JST-XH 2.5mm 2-pin | Vertical |
| 1 | J9 | UI breakout IDC (CHANGED v2.4) | IDC 2×10 box header 2.54mm | Vertical shrouded |
| 1 | J13 | USB-C | USB-C receptacle | SMD |
| 1 | J14 | PSU input | 3.5mm 2-pin screw terminal | Through-hole |
| 2 | SW_boot, SW_reset | Tactile switches | 6×6mm | SMD |
| 2 | LED_pwr, LED_stat | LEDs | Red + green 0805 | SMD |
| Note | — | J10/J11/J12 (three 3-pin button headers from v2.3) — REMOVED | — | — |

## 11.1 UI breakout board — separate BOM

| Qty | Component | Notes |
| --- | --- | --- |
| 1 | IDC 2×10 box header 2.54mm | Matches main board J9. Pin 1 marked on silkscreen. |
| 1 | 14-pin 2.54mm female socket | TFT module connector. Same 14-pin pinout as v2.3 Section 8.5. |
| 1 | EC11 rotary encoder 5-pin THT | Quadrature A/B + push switch SW + 2× GND/COM. |
| 2 | 6×6mm tactile push button THT | BTN1 (GPIO11) and BTN2 (GPIO12). |
| 1 | Passive piezo buzzer THT | 5V rated. Do NOT use active (self-oscillating) type. |
| 1 | 2N7002 N-ch MOSFET SOT-23 | Buzzer driver. Vgs(th) ≈1–2.5V, fully on at 3.3V gate drive. |
| 7 | 10kΩ 0402 resistors | EC11_A/B/SW pull-ups (×3), BTN1/BTN2 pull-ups (×2), buzzer gate (×1), TFT RST pull-up (×1). |
| 5 | 100nF X7R 0402 caps | Debounce: EC11_A, EC11_B, EC11_SW, BTN1, BTN2. |
| 1 | 47Ω 0402 resistor | TFT backlight LED series resistor. |
| 1 | 20-conductor IDC ribbon cable | Pre-crimped with IDC sockets at both ends. |

# 12. Pre-submission checklist (v2.4)

## 12.1 USB-C

- D+ net: GPIO20 → A6 AND B6 tied together. D− net: GPIO19 → A7 AND B7 tied together.

- CC1 and CC2: 5.1kΩ each to GND. SBU1 (A8) and SBU2 (B8): NC. SSTX/SSRX: NC.

## 12.2 Reverse polarity (AO4407A)

- Q1: Source (pins 2–4) → J14 V+. Drain (pins 5–8) → 24V rail. Gate (pin 1) → R_gate/D7 junction.

- D7 BZX84C15: cathode → Source, anode → Gate. Within 3mm of Q1. R_gate 10kΩ Gate → GND.

## 12.3 TMC2209

- R_UART 1kΩ: GPIO4 → junction. R_PDN_UP 10kΩ: junction → 3.3V. Both fitted.

- VCP 100nF per IC within 2mm. SPREAD 10kΩ pull-up per IC. EN 10kΩ pull-up per IC.

- VM 100µF + 100nF per IC. Thermal pad: GND continuity verified after reflow.

## 12.4 Motor connectors

- J4 and J5: JST-XH 6-pin. Pin order confirmed as 1:OA1 / 2:NC / 3:OA2 / 4:OB1 / 5:NC / 6:OB2.

- NC pins (2,5): tied to GND or confirmed NC. Pin 1 marked on silkscreen.

## 12.5 UAS and ADC

- C_adc 100nF at GPIO1 within 2mm of pad. C_env 10nF at envelope node (BAT54 cathode). Both present and distinct.

- IC1 ch-B tied off: non-inv (pin 5) to mid-rail, output (pin 7) to inv (pin 6).

## 12.6 IDC ribbon connector (J9)

- J9: IDC 2×10 shrouded box header with polarisation key. NOT a plain pin header.

- Pin 1 (GND, red stripe) marked on PCB silkscreen with triangle or dot.

- 100nF cap from IDC pin 12 (TOUCH_IRQ) trace to GND on main board — within 5mm of J9.

- J10/J11/J12 (old button headers from v2.3) confirmed absent from schematic and layout.

## 12.7 RPi UART (J3)

- J3: JST-XH 2.5mm 3-pin. Pin 1=GND, Pin 2=GPIO48 RX, Pin 3=GPIO47 TX. No power connection to RPi.

## 12.8 Firmware notes for designer

- ILI9341 SPI clock: reduce to 20MHz (was 40MHz) for ribbon cable signal integrity

- XPT2046 SPI clock: 2MHz max — firmware must reduce before each touch read

- AD9833 SPI mode: Mode 2 (CPOL=1) — firmware must switch from Mode 0 for display/touch

- GPIO13 BUZ_PWM: configure as LEDC output. Set frequency per desired tone before enabling.

- GPIO16/17 EC11 A/B: use hardware EXTI interrupt on both edges for quadrature decoding

- Touch calibration: store XPT2046 mapping coefficients in ESP32 NVS on first run

# 13. KiCad footprint reference (v2.4)

## 13.1 ICs and active components

| Ref | Part | KiCad footprint string |
| --- | --- | --- |
| U1 | ESP32-S3-WROOM-1-N8 | RF_Module:ESP32-S3-WROOM-1 |
| U2 | LM2596-5.0 | Package_TO_SOT_SMD:TO-263-5_TabPin3 |
| U3 | AMS1117-3.3 | Package_TO_SOT_SMD:SOT-223-3_TabPin2 |
| U4 | AD9833BRMZ | Package_SO:MSOP-10_3x3mm_P0.5mm |
| IC1, IC2 | OPA2354 (SOIC-8) | Package_SO:SOIC-8_3.9x4.9mm_P1.27mm |
| U5, U6 | TMC2209-LA (QFN-28) | Package_DFN_QFN:QFN-28-1EP_5x5mm_P0.5mm_EP3.7x3.7mm_ThermalVias |
| Q1 | AO4407A P-MOSFET (SOIC-8) | Package_SO:SOIC-8_3.9x4.9mm_P1.27mm |

## 13.2 Diodes and protection

| Ref | Part | KiCad footprint string |
| --- | --- | --- |
| D1, D3 | BAV99 | Diode_SMD:D_SOT-23 |
| D2 | BAT54 | Diode_SMD:D_SOD-123 |
| D4 | SS34A | Diode_SMD:D_SMA |
| D5 | SMBJ28A TVS | Diode_SMD:D_SMA |
| D6 | 1N5819 | Diode_SMD:D_SOD-123 |
| D7 | BZX84C15 15V Zener | Diode_SMD:D_SOT-23 |
| F1 | Polyfuse 1A 1812 | Fuse:Fuse_1812_4532Metric |

## 13.3 Inductor

| Ref | Part | KiCad footprint string |
| --- | --- | --- |
| L1 | 100µH ≥3A SMD | Inductor_SMD:L_Bourns-SRR1260 |

## 13.4 Resistors

| Group | Values | KiCad footprint string |
| --- | --- | --- |
| Standard 0402 | 10kΩ (×18 inc R_PDN_UP), 5.1kΩ (×2), 4.7kΩ, 1kΩ, 330Ω, 200Ω, 47Ω, 10Ω | Resistor_SMD:R_0402_1005Metric |
| Precision 1% 0402 | 390Ω (×1 Tx Rf), 910Ω (×2 Rx Rf), 100Ω (×3 Rg), 6.8kΩ (RSET) | Resistor_SMD:R_0402_1005Metric |
| Current sense 2512 | 0.11Ω 1% — RS1/RS2 (U5), RS3/RS4 (U6) | Resistor_SMD:R_2512_6332Metric |

## 13.5 Capacitors — ceramic 0402

| Value | Footprint | Qty / notes |
| --- | --- | --- |
| 100nF X7R | Capacitor_SMD:C_0402_1005Metric | 27 total — decoupling, debounce (limit switches), AC coupling, C_adc at GPIO1 |
| 10nF X7R | Capacitor_SMD:C_0402_1005Metric | 2 — C_env + C_COMP (AD9833) |
| 1nF X7R | Capacitor_SMD:C_0402_1005Metric | 1 — SW snubber |
| 1µF X7R | Capacitor_SMD:C_0402_1005Metric | 1 — C_en (ESP32 EN RC) |

## 13.6 Capacitors — 0805, tantalum, electrolytic (main board)

| Value / type | KiCad footprint string | Qty |
| --- | --- | --- |
| 10µF X5R 0805 | Capacitor_SMD:C_0805_2012Metric | 5 — ESP32 (×2), AD9833, IC1, IC2 |
| 10µF 10V tantalum Case A | Capacitor_Tantalum_SMD:CP_EIA-3216-18_Kemet-A | 2 — AMS1117 in/out |
| 100µF 50V electrolytic radial | Capacitor_THT:C_Radial_D8.0mm_P3.5mm | 1 — LM2596 input |
| 220µF 10V low-ESR electrolytic radial | Capacitor_THT:C_Radial_D8.0mm_P3.5mm | 1 — LM2596 output |
| 100µF 35V electrolytic radial | Capacitor_THT:C_Radial_D6.3mm_P2.5mm | 2 — TMC2209 VM (×1 each) |

## 13.7 Connectors — main board

| Ref | Description | KiCad footprint string |
| --- | --- | --- |
| J1, J2 | SMA PCB edge-launch | Connector_Coaxial:SMA_Molex_0732511150_EdgeMount |
| J3 | JST-XH 2.5mm 3-pin (RPi UART) | Connector_JST:JST_XH_B3B-XH-A_1x03_P2.50mm_Vertical |
| J4, J5 | JST-XH 2.5mm 6-pin (Motor 1 / Motor 2) | Connector_JST:JST_XH_B6B-XH-A_1x06_P2.50mm_Vertical |
| J6, J7 | JST-XH 2.5mm 2-pin (Limit switches) | Connector_JST:JST_XH_B2B-XH-A_1x02_P2.50mm_Vertical |
| J8 | JST-XH 2.5mm 2-pin (LED panel) | Connector_JST:JST_XH_B2B-XH-A_1x02_P2.50mm_Vertical |
| J9 | IDC 2×10 box header 2.54mm (UI breakout) | Connector_IDC:IDC-Header_2x10_P2.54mm_Vertical |
| J13 | USB-C receptacle | Connector_USB:USB_C_Receptacle_HRO_TYPE-C-31-M-12 |
| J14 | 3.5mm 2-pin screw terminal | TerminalBlock_Phoenix:TerminalBlock_Phoenix_PT-1,5-2-3.5_1x02_P3.50mm_Horizontal |

## 13.8 Breakout board footprints

| Component | KiCad footprint string |
| --- | --- |
| IDC 2×10 box header (breakout side) | Connector_IDC:IDC-Header_2x10_P2.54mm_Vertical |
| 14-pin 2.54mm female socket (TFT module) | Connector_PinHeader_2.54mm:PinSocket_1x14_P2.54mm_Vertical |
| EC11 rotary encoder 5-pin THT | Rotary_Encoder:RotaryEncoder_Alps_EC11E-Switch_Vertical_H20mm |
| 6×6mm tactile switch THT | Button_Switch_THT:SW_Tactile_Straight_KSA0Axx1LFTR |
| 2N7002 N-ch MOSFET (buzzer driver) | Package_TO_SOT_SMD:SOT-23 |
| All 0402 resistors on breakout | Resistor_SMD:R_0402_1005Metric |
| All 0402 caps on breakout | Capacitor_SMD:C_0402_1005Metric |

## 13.9 Switches and LEDs — main board

| Ref | Description | KiCad footprint string |
| --- | --- | --- |
| SW_boot, SW_reset | 6×6mm SMD tactile | Button_Switch_SMD:SW_SPST_EVQP2_6x6mm |
| LED_pwr | Red LED 0805 | LED_SMD:LED_0805_2012Metric |
| LED_stat | Green LED 0805 | LED_SMD:LED_0805_2012Metric |

— End of document —