EMBO

Embolization Particle Sizing System

Custom MCU Board — PCB Design Brief

| Field | Detail |
| --- | --- |
| Document version | v2.5 — TMC2209 migrated from QFN-28 hand-solder to BigTreeTech TMC2209 V1.x plug-in module (2×8 socket) |
| Date | June 2026 |
| Status | For schematic design |
| MCU module | ESP32-S3-WROOM-1-N8 (8MB flash, PCB antenna, no PSRAM) |
| Programming | USB-C direct GPIO19/20 — no USB-UART bridge chip needed |
| Wireless debug | Bluetooth LE 5.0 on-chip — NimBLE BLE UART |
| Motor drivers (CHANGED) | 2× BigTreeTech TMC2209 V1.x plug-in modules, socketed — field-replaceable, no QFN-28 hand-solder risk |
| Motor connectors | 2× JST-XH 6-pin — pin order A1/NC/A2/B1/NC/B2 |
| RPi UART | JST-XH 2.5mm 3-pin — GND(1)/RX(2)/TX(3) |
| UI system | Breakout board via 20-pin IDC ribbon (unchanged from v2.4): ILI9341 TFT, EC11 encoder, 2 buttons, passive piezo buzzer |
| Reverse polarity | AO4407A SOIC-8 P-MOSFET + BZX84C15 15V Zener gate clamp |
| Fabrication | JLCPCB — 2-layer FR4 ENIG, hot plate reflow with paste and stencil (AD9833 / OPA2354 only — TMC2209 no longer reflow-dependent) |

v2.5 changes from v2.4: U5/U6 TMC2209-LA QFN-28 (hand-soldered/reflowed) replaced by BigTreeTech TMC2209 V1.x plug-in modules in 2×8 0.1" sockets — eliminates the QFN-28 exposed thermal pad as a single point of failure and makes a bad driver field-replaceable. RS1–RS4 current sense resistors, VCP caps, and SPREAD pull-ups removed from main board BOM (now on-module or not applicable). SpreadCycle is now firmware-enforced via UART (GCONF.en_spreadcycle) since this module has no SPREAD pin. CLK pin corrected to GND tie (was incorrectly documented as floating in v2.1–v2.4). PDN pad confirmed as 4th pin down, left column, consistent across V1.1/V1.2/V1.3. UI breakout system (Section 8) unchanged from v2.4 — 20-pin IDC ribbon retained after evaluating and rejecting a 26-pin alternative.

# Revision history

| Version | Summary of change |
| --- | --- |
| v2.0 | STM32G474 → ESP32-S3-WROOM-1-N8. USB-C native programming. BLE debug. |
| v2.1 | UAS gain correction (Tx/Rx). TMC2209 VCP + SPREAD fixes. Power hardening. TFT module specified. |
| v2.2 | MOSFET AO3401A → AO4407A + BZX84C15 Zener gate clamp. Motor connectors 4×2-pin → 2×6-pin. KiCad footprint table added. |
| v2.3 | TFT confirmed ILI9341, 10-pin → 14-pin header. RPi UART JST-SH → JST-XH 3-pin. Motor pin order corrected. C_adc made explicit. TMC2209 PDN pull-up added. USB-C SBU/B-side pins documented. |
| v2.4 | UI system (TFT, EC11 encoder, 2 buttons, buzzer) moved to separate breakout board via 20-pin IDC ribbon cable. |
| v2.5 | TMC2209 QFN-28 hand-solder → BTT TMC2209 V1.x plug-in module sockets. SpreadCycle firmware-enforced. CLK pin correction. PDN pad confirmed. |

# 1. System overview

EMBO is a closed-loop embolic agent preparation device. The MCU board integrates three sensor subsystems (UAS, CV camera, StallGuard viscosity), two stepper motor actuators, two mechanical limit switches, and a UI breakout board connected via IDC ribbon cable.

## 1.1 Why ESP32-S3-WROOM-1-N8

- Module: chip, 8MB flash, 40MHz crystal, PCB antenna — no external MCU support components needed

- Built-in USB Serial/JTAG on GPIO19/20 — no bridge IC needed

- Bluetooth LE 5.0 on-chip — NimBLE for wireless debug

- 36 GPIO broken out — 28 used in this design, 8 free

- N8 variant: no PSRAM — GPIO35–37 free for SPI bus

## 1.2 Power rails

| Rail | Source | Voltage | Consumers | Max current |
| --- | --- | --- | --- | --- |
| 12/24V | PSU direct | 12–24V | 2× TMC2209 module VM | 6A peak |
| 5V | LM2596 buck | 5V ±2% | LED panel, TMC2209 module VCC_IO, buzzer | 2A |
| 3.3V | AMS1117-3.3 LDO | 3.3V ±1% | ESP32, AD9833, IC1/IC2, TFT logic | 800mA |

# 2. ESP32-S3-WROOM-1-N8 module

## 2.1 Footprint and keepout

- Dimensions: 18.0×25.5×3.1mm. Castellated pads, 1.27mm pitch.

- ANTENNA KEEPOUT: no copper, traces, vias, or components within 15mm of PCB antenna on ALL layers.

- EPAD: solder to GND pour.

## 2.2 Power decoupling

| Pin | Net | Decoupling |
| --- | --- | --- |
| VDD3P3 (×2) | 3.3V | 10µF X5R 0805 + 100nF X7R 0402 per pin — within 2mm |
| VDD3P3_RTC | 3.3V | 100nF X7R 0402 within 2mm |
| GND (multiple) | GND | All to ground pour |

## 2.3 EN reset + boot circuits

| Component | Value | Connection |
| --- | --- | --- |
| R_en | 10kΩ 0402 | 3.3V → EN pin |
| C_en | 1µF X7R 0402 | EN → GND |
| SW_reset | 6×6mm tactile | EN → GND. |
| R_boot | 10kΩ 0402 | 3.3V → GPIO0 |
| C_boot | 100nF X7R 0402 | GPIO0 → GND |
| SW_boot | 6×6mm tactile | GPIO0 → GND. |

## 2.4 USB-C programming connector (J13)

| USB-C signal | PCB connection | Notes |
| --- | --- | --- |
| D− A-side (A7) | GPIO19 | Tie A7+B7 together. |
| D− B-side (B7) | GPIO19 (same net) | Shorted on PCB. |
| D+ A-side (A6) | GPIO20 | Tie A6+B6 together. |
| D+ B-side (B6) | GPIO20 (same net) | Shorted on PCB. |
| CC1 (A5) | 5.1kΩ to GND | Mandatory. |
| CC2 (B5) | 5.1kΩ to GND | Mandatory. |
| VBUS | 5V rail via 1N5819 |  |
| GND | GND |  |
| SBU1 (A8) | NC |  |
| SBU2 (B8) | NC |  |
| SSTX / SSRX | NC |  |

# 3. Power section

## 3.1 Reverse polarity protection — AO4407A + BZX84C15 Zener

| Component | Value / Part | Connection |
| --- | --- | --- |
| Q1 | AO4407A SOIC-8 P-ch MOSFET −30V −12A | Source (2–4) → J14 V+. Drain (5–8) → protected 24V rail. Gate (1) → R_gate/D7 junction. |
| R_gate | 10kΩ 0402 | Gate → GND. |
| D7 | BZX84C15 15V Zener SOT-23 | Cathode → Source. Anode → Gate. Clamps Vgs=−15V. |

## 3.2 Input protection

| Component | Value | Connection |
| --- | --- | --- |
| F1 | 1A polyfuse | In series after Q1 |
| D5 | SMBJ28A TVS | Across 24V rail after F1 |
| Power LED | Red LED + 10kΩ | 3.3V → 10kΩ → LED → GND |

## 3.3 Buck: 24V → 5V (LM2596-5.0)

| Parameter | Requirement |
| --- | --- |
| IC | LM2596-5.0 fixed output |
| Input | 8–40V |
| Output | 5V 3A |
| Catch diode | SS34A — within 5mm of SW pin |
| SW snubber | 10Ω + 1nF |
| Bleed resistor | 4.7kΩ |

## 3.4 LDO: 5V → 3.3V (AMS1117-3.3)

| Parameter | Requirement |
| --- | --- |
| IC | AMS1117-3.3 SOT-223 |
| Output | 3.3V 800mA max |
| Thermal | SOT-223 tab to ≥100mm² copper + ≥4 vias. |

# 4. GPIO pin assignments

SPI mode: AD9833 Mode 2. ILI9341/XPT2046 Mode 0. XPT2046 max 2MHz. ILI9341 via ribbon: 20MHz.

| GPIO | Signal | Dir | Connected to | Notes |
| --- | --- | --- | --- | --- |
| 1 | UAS_ADC | Analog in | Envelope detector output | ADC1_CH0. 100nF C_adc within 2mm. |
| 2 | STATUS_LED | Out | 330Ω → green LED → GND |  |
| 4 | TMC_UART | Bidir | TMC2209 module PDN pin (pin 4) via junction | See Section 6.3. |
| 5 | STEP_M1 | Out | TMC2209 module U5 STEP | LEDC PWM |
| 6 | DIR_M1 | Out | module U5 DIR |  |
| 7 | EN_M1 | Out | module U5 EN | 10kΩ pull-up on main board |
| 8 | STEP_M2 | Out | module U6 STEP | LEDC PWM |
| 9 | DIR_M2 | Out | module U6 DIR |  |
| 10 | EN_M2 | Out | module U6 EN | 10kΩ pull-up on main board |
| 11 | BTN1 | In | IDC ribbon pin 16 | Breakout pull-up/debounce |
| 12 | BTN2 | In | IDC ribbon pin 17 | Breakout pull-up/debounce |
| 13 | BUZ_PWM | Out | IDC ribbon pin 18 | LEDC PWM |
| 14 | LIMIT_M1 | In | Limit switch 1 (J6) | 100nF debounce |
| 15 | LIMIT_M2 | In | Limit switch 2 (J7) | 100nF debounce |
| 16 | EC11_A | In | IDC ribbon pin 13 | Breakout pull-up/debounce |
| 17 | EC11_B | In | IDC ribbon pin 14 | Breakout pull-up/debounce |
| 18 | EC11_SW | In | IDC ribbon pin 15 | Breakout pull-up/debounce |
| 19 | USB D− | Fixed | USB-C J13 A7+B7 |  |
| 20 | USB D+ | Fixed | USB-C J13 A6+B6 |  |
| 21 | TOUCH_IRQ | In | IDC ribbon pin 12 | 100nF near J9 |
| 35 | SPI_MOSI | Out | AD9833 + IDC ribbon pin 5 | SPI2 |
| 36 | SPI_CLK | Out | AD9833 + IDC ribbon pin 7 | SPI2, 20MHz via ribbon |
| 37 | SPI_MISO | In | IDC ribbon pin 6 | SPI2 |
| 38 | AD9833_CS | Out | AD9833 FSYNC |  |
| 39 | TFT_CS | Out | IDC ribbon pin 8 |  |
| 40 | TFT_DC | Out | IDC ribbon pin 9 |  |
| 41 | TFT_RST | Out | IDC ribbon pin 10 |  |
| 42 | TOUCH_CS | Out | IDC ribbon pin 11 |  |
| 47 | RPi_TX | Out | J3 pin 3 | UART1 TX 921600 baud |
| 48 | RPi_RX | In | J3 pin 2 | UART1 RX |

# 5. UAS signal chain

Unchanged from v2.4. 1MHz acoustic chain on 3.3V rail.

## 5.1 AD9833BRMZ

| Parameter | Requirement |
| --- | --- |
| IC | AD9833BRMZ MSOP-10 |
| Supply | 3.3V |
| SPI | GPIO38/36/35, Mode 2 |
| RSET | 6.8kΩ 1% → 604mVpp |
| COMP | 10nF |

## 5.2 Tx amplifier — IC1 ch-A, G=4.9

| Parameter | Requirement |
| --- | --- |
| IC | OPA2354 ch-A IC1 |
| Gain | Rf=390Ω, Rg=100Ω. G=4.9. Output 2.96Vpp. |

## 5.3 IC1 ch-B — tied off

- Non-inv to mid-rail. Inv to output — unity-gain follower.

## 5.4 Rx protection

| Component | Value | Function |
| --- | --- | --- |
| R_in | 100Ω | Series limiter |
| D1 | BAV99 | Clamp 0–3.3V |

## 5.5 Rx amplification — G=100

| Stage | Pin | Rf | Rg | Gain |
| --- | --- | --- | --- | --- |
| Stage 1 | IC2 ch-A | 910Ω | 100Ω | 10.1× |
| Stage 2 | IC2 ch-B | 910Ω | 100Ω | 10.1× |

## 5.6 Envelope and ADC

| Component | Value | Function |
| --- | --- | --- |
| D2 | BAT54 | Rectifier |
| R_env | 10kΩ | τ=100µs |
| C_env | 10nF | RC filter |
| D3 | BAV99 | GPIO1 clamp |
| C_adc | 100nF | GPIO1 noise filter, within 2mm |

# 6. Motor drivers — 2× BigTreeTech TMC2209 V1.x plug-in modules

U5/U6 changed from TMC2209-LA QFN-28 to BTT TMC2209 V1.x plug-in modules in 2×8 sockets. Eliminates QFN-28 thermal pad as single point of failure; field-replaceable.

FUNCTIONAL SENSOR INPUT: StallGuard SG_RESULT feeds the PID mixing-stop algorithm — affects the science, not just motion. Depends on sense resistor, coil current, and SpreadCycle mode.

## 6.1 Module specifications

- Fixed onboard sense resistor: 110mΩ 1%, confirmed via BTT docs, consistent V1.1–V1.3. Same class as original RS1–RS4 — scaling math unaffected.

- Current set via onboard trimmer pot (VREF) by default — main source of run-to-run variability vs original design.

- DIAG/INDEX broken out but NOT used — viscosity sensing via UART-polled SG_RESULT.

## 6.2 UART addressing

| Driver | Ref | MS1 | MS2 | Address |
| --- | --- | --- | --- | --- |
| Motor 1 | U5 | GND | GND | 0 |
| Motor 2 | U6 | 3.3V via 10kΩ | GND | 1 |

## 6.3 PDN_UART pad — confirmed location

VERIFIED: all BTT TMC2209 module variants (V1.1, V1.2, V1.3) route the active UART signal out of the 4th pin down (left column), labeled PDN, by factory default. The 5th pin (adjacent) is a spare PDN pad, not a duplicate — route only the 4th pin.

| Component | Value | Connection |
| --- | --- | --- |
| R_UART | 1kΩ | GPIO4 → PDN junction |
| R_PDN_UP | 10kΩ | PDN junction → 3.3V |

## 6.4 SpreadCycle — firmware-enforced

CRITICAL CHANGE: this module has NO SPREAD pin. SPREAD pull-up resistors REMOVED from BOM entirely.

SpreadCycle required for StallGuard4. Firmware MUST force via UART at every boot:

- Write GCONF.en_spreadcycle=1 over UART to both U5 and U6

- Read back DRVSTATUS/GCONF to confirm the write succeeded

- HARDWARE VERIFICATION GATE: manual load test on real module, confirm SG_RESULT responds before trusting data for PID tuning

## 6.5 CLK pin — correction from v2.1–v2.4

CORRECTION: prior versions incorrectly stated CLK floats for internal oscillator. Per datasheet, CLK must tie to GND.

- CLK (6th pin down, left column): tie to GND via 0Ω resistor or direct trace — 2 total

## 6.6 VDD pin — unverified

NOT CONFIRMED: 3.3V assumed safe for module VDD but not verified against specific module schematic. Confirm before powering on.

## 6.7 Pin mapping — module to main board

| Module pin (left col, top→bottom) | Net | Notes |
| --- | --- | --- |
| 1. EN | GPIO7(U5)/GPIO10(U6) | Explicit 10kΩ pull-up — do not leave NC |
| 2. MS1 | GND(U5)/3.3V via 10kΩ(U6) |  |
| 3. MS2 | GND |  |
| 4. PDN (factory-wired) | UART junction | CONFIRMED active pad |
| 5. PDN (spare) | NC |  |
| 6. CLK | GND | NEW correction |
| 7. STEP | GPIO5(U5)/GPIO8(U6) |  |
| 8. DIR | GPIO6(U5)/GPIO9(U6) |  |

| Module pin (right col) | Net | Notes |
| --- | --- | --- |
| VM | 24V + local bulk caps |  |
| GND | GND pour |  |
| A2 | J4/J5 | CAUTION: order is VM,GND,A2,A1,B1,B2,VDD,GND — A2 precedes A1 |
| A1 | J4/J5 | See caution above |
| B1 | J4/J5 |  |
| B2 | J4/J5 |  |
| VDD | 3.3V rail | Replaces VCC_IO, see Section 6.6 |
| GND | GND pour |  |

| Module top pins | Net | Notes |
| --- | --- | --- |
| DIAG | NC | Not used |
| INDEX | NC | Not used |
| VREF | Onboard trimpot | Firmware: GCONF.I_scale_analog=0, use UART IHOLD_IRUN instead |

## 6.8 Mechanical note

Module + socket stack taller than QFN-28. Confirm enclosure clearance. Sockets vibration-exposed — recommend good pin-retention or adhesive dab.

# 7. Limit switches

| GPIO | Ref | Connector | Wiring |
| --- | --- | --- | --- |
| GPIO14 | LIMIT_M1 | J6 | Pin 1 GND, Pin 2 GPIO14, 100nF debounce |
| GPIO15 | LIMIT_M2 | J7 | Pin 1 GND, Pin 2 GPIO15, 100nF debounce |

# 8. UI interface system — breakout board via IDC ribbon

Unchanged from v2.4. A 26-pin IDC alternative was evaluated and rejected in favor of this simpler 20-pin layout.

TFT, EC11 encoder, 2 buttons, buzzer on separate breakout PCB via 20-conductor IDC ribbon. Touch and display SPI tied together on the breakout.

## 8.1 IDC ribbon — 20-pin 2×10, odd-even

| Pin | Signal | GPIO/net | Notes |
| --- | --- | --- | --- |
| 1 | GND | GND | Red stripe ref |
| 2 | 3.3V | 3.3V rail |  |
| 3 | GND | GND |  |
| 4 | 5V | 5V rail | Buzzer |
| 5 | SPI_MOSI | GPIO35 | Shared on breakout |
| 6 | SPI_MISO | GPIO37 | Shared |
| 7 | SPI_CLK | GPIO36 | 20MHz max via ribbon |
| 8 | TFT_CS | GPIO39 |  |
| 9 | TFT_DC | GPIO40 |  |
| 10 | TFT_RST | GPIO41 |  |
| 11 | TOUCH_CS | GPIO42 |  |
| 12 | TOUCH_IRQ | GPIO21 | 100nF near J9 |
| 13 | EC11_A | GPIO16 | Breakout pull-up |
| 14 | EC11_B | GPIO17 | Breakout pull-up |
| 15 | EC11_SW | GPIO18 | Breakout pull-up |
| 16 | BTN1 | GPIO11 | Breakout pull-up |
| 17 | BTN2 | GPIO12 | Breakout pull-up |
| 18 | BUZ_PWM | GPIO13 | 2N7002 gate |
| 19 | GND | GND |  |
| 20 | NC | — | Spare |

## 8.2 SPI signal integrity

| Device | SPI mode | Clock via ribbon |
| --- | --- | --- |
| ILI9341 | Mode 0 | 20MHz max |
| XPT2046 | Mode 0 | 2MHz |
| AD9833 | Mode 2 | ~10MHz, direct trace |

## 8.3 Breakout BOM

| Component | Qty |
| --- | --- |
| IDC 2×10 box header | 1 |
| 14-pin TFT socket | 1 |
| EC11 encoder | 1 |
| Tactile buttons | 2 |
| Passive piezo buzzer | 1 |
| 2N7002 MOSFET | 1 |
| 10kΩ 0402 | 7 |
| 100nF 0402 | 5 |
| 47Ω 0402 | 1 |

## 8.4 TFT connection (14-pin)

| Pin | Label | Signal |
| --- | --- | --- |
| 1 | VCC | Ribbon 2 (3.3V) |
| 2 | GND | Ribbon GND |
| 3 | CS | Ribbon 8 |
| 4 | RESET | Ribbon 10, 10kΩ pull-up |
| 5 | DC | Ribbon 9 |
| 6 | SDI | Ribbon 5 |
| 7 | SCK | Ribbon 7 |
| 8 | LED | 3.3V via 47Ω local |
| 9 | SDO | Ribbon 6 |
| 10 | T_CLK | Tied to pin 7 |
| 11 | T_CS | Ribbon 11 |
| 12 | T_DIN | Tied to pin 6 |
| 13 | T_DO | Tied to pin 9 |
| 14 | T_IRQ | Ribbon 12 |

## 8.5 EC11 and buttons

- EC11 A/B/SW → ribbon 13/14/15. Each: 10kΩ pull-up + 100nF debounce on breakout.

- BTN1/BTN2 → ribbon 16/17. Same treatment.

## 8.6 Buzzer driver

| Component | Connection |
| --- | --- |
| Buzzer | + → ribbon 5V. − → 2N7002 drain. |
| 2N7002 | Gate → 10kΩ → ribbon BUZ_PWM. Source → GND. |

# 9. External connectors — complete list (v2.5)

| Ref | Label | Type | Pin assignment |
| --- | --- | --- | --- |
| J1 | TX Transducer | SMA edge-launch | IC1 ch-A output. Shell AGND. |
| J2 | RX Transducer | SMA edge-launch | R_in→D1. Shell AGND. |
| J3 | RPi UART | JST-XH 3-pin | GND/RX/TX, 921600 baud |
| J4 | Motor 1 | JST-XH 6-pin | 1:OA1 2:NC 3:OA2 4:OB1 5:NC 6:OB2 — U5 module |
| J5 | Motor 2 | JST-XH 6-pin | 1:OA1 2:NC 3:OA2 4:OB1 5:NC 6:OB2 — U6 module |
| J6 | Limit M1 | JST-XH 2-pin | GND/GPIO14 |
| J7 | Limit M2 | JST-XH 2-pin | GND/GPIO15 |
| J8 | LED panel | JST-XH 2-pin | 5V/GND |
| J9 | UI breakout IDC | IDC 2×10 | 20-pin, unchanged from v2.4 |
| J13 | USB-C | USB-C receptacle | A6+B6→GPIO20, A7+B7→GPIO19 |
| J14 | PSU input | Screw terminal | Via Q1→F1→D5 |
| SK1, SK2 | TMC2209 module sockets (NEW) | 2×8 0.1" pin socket | U5/U6 module mounting |

# 10. PCB layout rules

## 10.1 Layer stack

- 2-layer FR4 ENIG. UAS analog top layer only, no vias.

## 10.2 Antenna keepout

- No copper within 15mm of ESP32 antenna on ALL layers.

## 10.3 Power section

- Q1/D7 within 3mm. SS34A within 5mm of SW pin. AMS1117 thermal vias mandatory.

## 10.4 UAS chain

- IC1/IC2 ≥15mm separation. C_adc within 2mm of GPIO1.

## 10.5 TMC2209 module sockets (CHANGED)

- SK1, SK2: 2×8 0.1" sockets, hand-solderable — no reflow dependency for these positions.

- Hot-plate reflow requirement previously specific to QFN-28 no longer applies to U5/U6. Confirm separately for AD9833/OPA2354.

- VM bulk caps remain on main board near each socket.

- Confirm enclosure clearance for module+socket height with mechanical team.

## 10.6 IDC ribbon (J9) — unchanged

- 100nF cap from TOUCH_IRQ trace near J9.

## 10.7 USB-C routing

- D+/D− as single nets across A/B pads. CC resistors within 5mm.

# 11. Bill of materials — main board (v2.5)

| Qty | Ref | Description | Part | Package |
| --- | --- | --- | --- | --- |
| 1 | U1 | ESP32-S3 module | ESP32-S3-WROOM-1-N8 | Castellated |
| 1 | Q1 | P-MOSFET | AO4407A | SOIC-8 |
| 1 | D7 | Zener clamp | BZX84C15 | SOT-23 |
| 1 | U2 | Buck | LM2596-5.0 | TO-263-5 |
| 1 | U3 | LDO | AMS1117-3.3 | SOT-223 |
| 1 | U4 | DDS gen | AD9833BRMZ | MSOP-10 |
| 2 | IC1, IC2 | Op-amp | OPA2354 | SOIC-8 |
| 2 | SK1, SK2 | Module socket (CHANGED) | 2×8 0.1" pin socket | THT |
| 2 | — | TMC2209 module (CHANGED) | BTT TMC2209 V1.x | Plug-in |
| 2 | D1, D3 | Diode clamp | BAV99 | SOT-23 |
| 1 | D2 | Rectifier | BAT54 | SOD-123 |
| 1 | D4 | Catch diode | SS34A | SMA |
| 1 | D5 | TVS | SMBJ28A | SMA |
| 1 | D6 | Schottky | 1N5819 | SOD-123 |
| 1 | F1 | Polyfuse | 1A | 1812 |
| 1 | L1 | Inductor | 100µH | SMD |
| 2 | J1, J2 | SMA | — | Board edge |
| 1 | J3 | RPi UART | JST-XH 3-pin |  |
| 2 | J4, J5 | Motors | JST-XH 6-pin |  |
| 2 | J6, J7 | Limits | JST-XH 2-pin |  |
| 1 | J8 | LED | JST-XH 2-pin |  |
| 1 | J9 | IDC | 2×10 box header |  |
| 1 | J13 | USB-C | — | SMD |
| 1 | J14 | PSU | Screw terminal |  |
| 2 | SW_boot, SW_reset | Tactile | 6×6mm |  |
| 2 | LED_pwr, LED_stat | LEDs | 0805 |  |

## 11.1 Removed from BOM in v2.5

| Item | Reason |
| --- | --- |
| U5, U6 QFN-28 footprint | Replaced by sockets SK1/SK2 |
| RS1–RS4 current sense | Fixed sense resistor on-module |
| VCP caps (×2) | On-module |
| SPREAD pull-ups (×2) | No SPREAD pin — firmware-enforced |

## 11.2 Breakout BOM (unchanged)

| Qty | Component |
| --- | --- |
| 1 | IDC 2×10 box header |
| 1 | 14-pin TFT socket |
| 1 | EC11 encoder |
| 2 | Tactile buttons |
| 1 | Passive buzzer |
| 1 | 2N7002 |
| 7 | 10kΩ 0402 |
| 5 | 100nF 0402 |
| 1 | 20-cond IDC ribbon cable |

# 12. Pre-submission checklist (v2.5)

## 12.1 TMC2209 module — NEW

- SK1, SK2 placed correctly, module key/notch alignment confirmed

UNVERIFIED: module VDD tolerance against purchased module datasheet

- PDN: only 4th pin (left col) routed. 5th pin confirmed NC.

- CLK tied to GND — not floating.

- EN: explicit 10kΩ pull-up on main board, both modules.

- SPREAD pull-ups confirmed ABSENT.

- RS1–RS4, VCP caps confirmed ABSENT.

FIRMWARE GATE: GCONF.en_spreadcycle=1 + readback at boot. Load test confirms SG_RESULT response before PID tuning.

- A1/A2 wiring confirmed against actual module pin order, not assumed.

- Mechanical clearance confirmed.

## 12.2 USB-C

- D+ A6+B6, D− A7+B7. CC1/CC2 fitted. SBU NC.

## 12.3 Reverse polarity

- Q1/D7 wiring confirmed per Section 3.1.

## 12.4 Motor connectors

- J4/J5 pin order confirmed: 1:OA1 2:NC 3:OA2 4:OB1 5:NC 6:OB2.

## 12.5 UAS and ADC

- C_adc and C_env present and distinct. IC1 ch-B tied off.

## 12.6 UI breakout / IDC

- J9 pin 1 marked. 100nF at TOUCH_IRQ near J9.

## 12.7 Firmware notes

- SpreadCycle + readback. I_scale_analog=0 + IHOLD_IRUN.

- ILI9341 20MHz, XPT2046 2MHz, AD9833 Mode 2.

# 13. KiCad footprint reference (v2.5)

## 13.1 ICs and active components

| Ref | Part | KiCad footprint string |
| --- | --- | --- |
| U1 | ESP32-S3-WROOM-1-N8 | RF_Module:ESP32-S3-WROOM-1 |
| U2 | LM2596-5.0 | Package_TO_SOT_SMD:TO-263-5_TabPin3 |
| U3 | AMS1117-3.3 | Package_TO_SOT_SMD:SOT-223-3_TabPin2 |
| U4 | AD9833BRMZ | Package_SO:MSOP-10_3x3mm_P0.5mm |
| IC1, IC2 | OPA2354 | Package_SO:SOIC-8_3.9x4.9mm_P1.27mm |
| SK1, SK2 | Module socket (CHANGED) | Connector_PinSocket_2.54mm:PinSocket_2x08_P2.54mm_Vertical |
| Q1 | AO4407A | Package_SO:SOIC-8_3.9x4.9mm_P1.27mm |

U5/U6 QFN-28 footprint REMOVED in v2.5 — replaced by SK1/SK2 above.

## 13.2 Diodes and protection

| Ref | Part | KiCad footprint string |
| --- | --- | --- |
| D1, D3 | BAV99 | Diode_SMD:D_SOT-23 |
| D2 | BAT54 | Diode_SMD:D_SOD-123 |
| D4 | SS34A | Diode_SMD:D_SMA |
| D5 | SMBJ28A | Diode_SMD:D_SMA |
| D6 | 1N5819 | Diode_SMD:D_SOD-123 |
| D7 | BZX84C15 | Diode_SMD:D_SOT-23 |
| F1 | Polyfuse | Fuse:Fuse_1812_4532Metric |

## 13.3 Inductor

| Ref | Part | KiCad footprint string |
| --- | --- | --- |
| L1 | 100µH ≥3A | Inductor_SMD:L_Bourns-SRR1260 |

## 13.4 Resistors

| Group | Values | KiCad footprint string |
| --- | --- | --- |
| Standard 0402 | 10kΩ (×16), 5.1kΩ(×2), 4.7kΩ, 1kΩ, 330Ω, 200Ω, 47Ω, 10Ω, 0Ω(CLK×2) | Resistor_SMD:R_0402_1005Metric |
| Precision 1% | 390Ω, 910Ω(×2), 100Ω(×3), 6.8kΩ | Resistor_SMD:R_0402_1005Metric |

RS1–RS4 (2512) REMOVED in v2.5.

## 13.5 Capacitors — ceramic 0402

| Value | Footprint | Qty |
| --- | --- | --- |
| 100nF X7R | Capacitor_SMD:C_0402_1005Metric | 25 (2 fewer than v2.4) |
| 10nF X7R | Capacitor_SMD:C_0402_1005Metric | 2 |
| 1nF X7R | Capacitor_SMD:C_0402_1005Metric | 1 |
| 1µF X7R | Capacitor_SMD:C_0402_1005Metric | 1 |

## 13.6 Capacitors — 0805, tantalum, electrolytic

| Value/type | KiCad footprint string | Qty |
| --- | --- | --- |
| 10µF X5R 0805 | Capacitor_SMD:C_0805_2012Metric | 5 |
| 10µF tantalum Case A | Capacitor_Tantalum_SMD:CP_EIA-3216-18_Kemet-A | 2 |
| 100µF radial | Capacitor_THT:C_Radial_D8.0mm_P3.5mm | 1 |
| 220µF radial | Capacitor_THT:C_Radial_D8.0mm_P3.5mm | 1 |
| 100µF radial (TMC sockets) | Capacitor_THT:C_Radial_D6.3mm_P2.5mm | 2 |

## 13.7 Connectors — main board

| Ref | Description | KiCad footprint string |
| --- | --- | --- |
| J1, J2 | SMA edge-launch | Connector_Coaxial:SMA_Molex_0732511150_EdgeMount |
| J3 | JST-XH 3-pin | Connector_JST:JST_XH_B3B-XH-A_1x03_P2.50mm_Vertical |
| J4, J5 | JST-XH 6-pin | Connector_JST:JST_XH_B6B-XH-A_1x06_P2.50mm_Vertical |
| J6, J7 | JST-XH 2-pin | Connector_JST:JST_XH_B2B-XH-A_1x02_P2.50mm_Vertical |
| J8 | JST-XH 2-pin | Connector_JST:JST_XH_B2B-XH-A_1x02_P2.50mm_Vertical |
| J9 | IDC 2×10 | Connector_IDC:IDC-Header_2x10_P2.54mm_Vertical |
| J13 | USB-C | Connector_USB:USB_C_Receptacle_HRO_TYPE-C-31-M-12 |
| J14 | Screw terminal | TerminalBlock_Phoenix:TerminalBlock_Phoenix_PT-1,5-2-3.5_1x02_P3.50mm_Horizontal |
| SK1, SK2 | Module sockets (NEW) | Connector_PinSocket_2.54mm:PinSocket_2x08_P2.54mm_Vertical |

## 13.8 Breakout board footprints (unchanged)

| Component | KiCad footprint string |
| --- | --- |
| IDC 2×10 box header | Connector_IDC:IDC-Header_2x10_P2.54mm_Vertical |
| 14-pin TFT socket | Connector_PinHeader_2.54mm:PinSocket_1x14_P2.54mm_Vertical |
| EC11 encoder | Rotary_Encoder:RotaryEncoder_Alps_EC11E-Switch_Vertical_H20mm |
| 6×6mm tactile switch | Button_Switch_THT:SW_Tactile_Straight_KSA0Axx1LFTR |
| 2N7002 | Package_TO_SOT_SMD:SOT-23 |

## 13.9 Switches and LEDs — main board

| Ref | Description | KiCad footprint string |
| --- | --- | --- |
| SW_boot, SW_reset | 6×6mm SMD tactile | Button_Switch_SMD:SW_SPST_EVQP2_6x6mm |
| LED_pwr | Red LED 0805 | LED_SMD:LED_0805_2012Metric |
| LED_stat | Green LED 0805 | LED_SMD:LED_0805_2012Metric |

— End of document —