EMBO

Embolization Particle Sizing System

Custom MCU Board — PCB Design Brief

| Field | Detail |
| --- | --- |
| Document version | v2.2 — AO4407A MOSFET + Zener gate clamp, 6-pin motor connectors, KiCad footprint table |
| Date | June 2026 |
| Status | For schematic design |
| MCU module | ESP32-S3-WROOM-1-N8 (8MB flash, PCB antenna, no PSRAM) |
| Programming | USB-C direct GPIO19/20 — no USB-UART bridge chip needed |
| Wireless debug | Bluetooth LE 5.0 on-chip — NimBLE BLE UART for wireless monitoring |
| Motor drivers | 2× TMC2209-LA QFN-28 — shared UART GPIO4, StallGuard viscosity sensing |
| Motor connectors | 2× JST-XH 6-pin (pins 1–4 used, pins 5–6 NC) — one per motor |
| Reverse polarity | AO4407A SOIC-8 P-MOSFET + BZX84C15 15V Zener gate clamp |
| Fabrication | JLCPCB — 2-layer FR4 ENIG, hot plate reflow with paste and stencil |

v2.2 changes from v2.1: (1) Reverse polarity guard changed from AO3401A (SOT-23) to AO4407A (SOIC-8, higher current rating). BZX84C15 15V Zener (D7) added as gate-source clamp — without it, Vgs = -24V exceeds AO4407A ±20V Vgs_max. (2) Four 2-pin motor coil connectors (J4–J7) replaced by two 6-pin connectors (J4 Motor 1, J5 Motor 2), pins 1–4 used, 5–6 NC. Connector numbering updated throughout. (3) KiCad footprint reference table added as Section 13.

# 1. System overview

EMBO is a closed-loop embolic agent preparation device. The MCU board integrates three sensor subsystems (UAS, CV camera, StallGuard viscosity), two stepper motor actuators, two mechanical limit switches, and a touch-screen UI — coordinated by an ESP32-S3 communicating with a Raspberry Pi 5 via UART.

## 1.1 Why ESP32-S3-WROOM-1-N8

- Module integrates chip, 8MB flash, 40MHz crystal, PCB antenna — no external MCU support components needed

- Built-in USB Serial/JTAG on GPIO19/20 — program via USB-C, no bridge IC needed

- Bluetooth LE 5.0 on-chip — NimBLE library for wireless debug stream

- 36 GPIO broken out — 25 used, 11 free

- N8 variant: no PSRAM — GPIO35–37 free for SPI bus

## 1.2 Power rails

| Rail | Source | Voltage | Consumers | Max current |
| --- | --- | --- | --- | --- |
| 12/24V | PSU direct | 12–24V | 2× TMC2209 VM | 6A peak |
| 5V | LM2596 buck | 5V ±2% | LED panel, TFT, TMC2209 VCC_IO | 2A |
| 3.3V | AMS1117-3.3 LDO | 3.3V ±1% | ESP32 module, AD9833, IC1/IC2, BAV99, BAT54, TFT logic | 800mA |

# 2. ESP32-S3-WROOM-1-N8 module

## 2.1 Footprint and keepout

- Dimensions: 18.0 × 25.5 × 3.1mm. Castellated pads, 1.27mm pitch.

- ANTENNA KEEPOUT: no copper, traces, vias, or components within 15mm of PCB antenna on ALL layers including ground plane.

- EPAD (bottom): solder to GND pour.

## 2.2 Power decoupling

| Pin | Net | Decoupling |
| --- | --- | --- |
| VDD3P3 (×2) | 3.3V | 10µF X5R 0805 + 100nF X7R 0402 per pin — within 2mm |
| VDD3P3_RTC | 3.3V | 100nF X7R 0402 within 2mm |
| GND (multiple) | GND | All to ground pour |

## 2.3 EN reset + boot circuits

| Component | Value | Connection |
| --- | --- | --- |
| R_en | 10kΩ 0402 | 3.3V → EN pin |
| C_en | 1µF X7R 0402 | EN pin → GND |
| SW_reset | 6×6mm tactile | EN pin → GND |
| R_boot | 10kΩ 0402 | 3.3V → GPIO0 |
| C_boot | 100nF X7R 0402 | GPIO0 → GND |
| SW_boot | 6×6mm tactile | GPIO0 → GND |

## 2.4 USB-C programming connector

| USB-C signal | Connection | Notes |
| --- | --- | --- |
| D− | GPIO19 | Direct — fixed USB PHY, no other connection |
| D+ | GPIO20 | Direct — fixed USB PHY, no other connection |
| VBUS | 5V rail via 1N5819 | Allows USB-only power during development |
| CC1, CC2 | 5.1kΩ each to GND | Mandatory for USB-C charger detection |

# 3. Power section

## 3.1 Reverse polarity protection — AO4407A P-channel MOSFET

Changed from AO3401A (SOT-23) in v2.1 to AO4407A (SOIC-8) for higher current handling. The AO4407A has Vgs_max = ±20V. With a 24V supply and gate pulled directly to GND, Vgs = −24V which exceeds this limit. A 15V Zener diode (D7) clamps the gate-source voltage to a safe −15V.

| Component | Value / Part | Connection |
| --- | --- | --- |
| Q1 | AO4407A SOIC-8 P-channel MOSFET, −30V, −12A | Source (pins 2–4) → J14 V+. Drain (pins 5–8) → protected 24V rail. Gate (pin 1) → R_gate / D7 junction. |
| R_gate | 10kΩ 0402 | Gate → GND. Ensures MOSFET turns off when input is disconnected. |
| D7 | BZX84C15 15V Zener SOT-23 | Cathode → Source (24V+ line). Anode → Gate junction. Clamps Vgs to −15V during normal operation. Without D7, Vgs = −24V exceeds AO4407A ±20V Vgs_max. |

Operation: normal polarity — Gate is clamped to (24V − 15V) = 9V by D7, giving Vgs = 9 − 24 = −15V. P-channel turns ON. Reverse polarity — Source becomes negative, Gate is pulled to circuit GND (now the positive rail), Vgs becomes positive, P-channel stays OFF. Circuit protected.

## 3.2 Input protection (after Q1)

| Component | Value | Connection |
| --- | --- | --- |
| F1 | 1A polyfuse (1812) | In series on 24V rail after Q1 |
| D5 | SMBJ28A TVS | Across 24V rail after F1 — clamps transients |
| Power LED | Red LED + 10kΩ 0402 | 10kΩ from 3.3V to GND via LED |

## 3.3 Buck: 24V → 5V (LM2596-5.0)

| Parameter | Requirement |
| --- | --- |
| IC | LM2596-5.0 fixed output |
| Input range | 8–40V |
| Output | 5V, 3A |
| Inductor | 100µH ≥3A saturation, low DCR |
| Input cap | 100µF electrolytic ≥40V + 100nF ceramic |
| Output cap | 220µF low-ESR electrolytic + 100nF ceramic |
| Catch diode | SS34A (DO-214AB SMA) — SMD substitute for 1N5822. Place within 5mm of SW pin. |
| SW snubber | 10Ω + 1nF from SW pin to GND |
| Bleed resistor | 4.7kΩ from 5V to GND |

## 3.4 LDO: 5V → 3.3V (AMS1117-3.3)

| Parameter | Requirement |
| --- | --- |
| IC | AMS1117-3.3 SOT-223 |
| Output | 3.3V, 800mA max |
| Input cap | 10µF tantalum (required for stability) |
| Output cap | 10µF tantalum + 100nF ceramic |
| Thermal | CRITICAL: SOT-223 tab → ≥100mm² copper pour + ≥4 thermal vias. Dissipation ≈850mW at max load. |

# 4. GPIO pin assignments

SPI BUS MODE: AD9833 uses SPI Mode 2 (CPOL=1, CPHA=0). ILI9341/ST7789V and XPT2046 use SPI Mode 0 (CPOL=0, CPHA=0). Firmware must switch SPI mode between AD9833 and display/touch transactions. XPT2046 max SPI clock: 2MHz — reduce before touch reads, restore to 40MHz for display.

| GPIO | Signal | Dir | Connected to | Notes |
| --- | --- | --- | --- | --- |
| 1 | UAS_ADC | Analog in | Envelope detector output (direct, no divider) | ADC1_CH0 — must be ADC1, safe with BLE. 100nF to GND. |
| 2 | STATUS_LED | Out | 330Ω → green LED → GND | GPIO out |
| 4 | TMC_UART | Bidir | Both TMC2209 PDN_UART via 1kΩ | UART1 TX and RX both mapped to GPIO4 (half-duplex). 1kΩ series to junction. |
| 5 | STEP_M1 | Out | TMC2209 U5 STEP | LEDC PWM |
| 6 | DIR_M1 | Out | TMC2209 U5 DIR | GPIO out |
| 7 | EN_M1 | Out | TMC2209 U5 EN (active LOW) | 10kΩ pull-up to 3.3V at IC |
| 8 | STEP_M2 | Out | TMC2209 U6 STEP | LEDC PWM |
| 9 | DIR_M2 | Out | TMC2209 U6 DIR | GPIO out |
| 10 | EN_M2 | Out | TMC2209 U6 EN (active LOW) | 10kΩ pull-up to 3.3V at IC |
| 14 | LIMIT_M1 | In | Limit switch 1 → GND (J6) | Internal pull-up. 100nF debounce. |
| 15 | LIMIT_M2 | In | Limit switch 2 → GND (J7) | Internal pull-up. 100nF debounce. |
| 16 | BTN_UP | In | Tactile switch → GND | Internal pull-up. 100nF debounce. |
| 17 | BTN_DOWN | In | Tactile switch → GND | Internal pull-up. 100nF debounce. |
| 18 | BTN_SELECT | In | Tactile switch → GND | Internal pull-up. 100nF debounce. |
| 19 | USB D− | Fixed | USB-C J13 D− | Fixed USB PHY — no other connection |
| 20 | USB D+ | Fixed | USB-C J13 D+ | Fixed USB PHY — no other connection |
| 21 | TOUCH_IRQ | In | XPT2046 T_IRQ | Internal pull-up. 100nF to GND. |
| 35 | SPI_MOSI | Out | AD9833 SDATA + TFT MOSI | SPI2. Mode 0 for display/touch, Mode 2 for AD9833. |
| 36 | SPI_CLK | Out | AD9833 SCLK + TFT SCL | SPI2. 40MHz display, 2MHz touch. |
| 37 | SPI_MISO | In | TFT SDO + T_DO | SPI2. Touch readback only. |
| 38 | AD9833_CS | Out | AD9833 FSYNC (active low) | GPIO out |
| 39 | TFT_CS | Out | ILI9341/ST7789V CS | GPIO out |
| 40 | TFT_DC | Out | ILI9341/ST7789V DC | GPIO out |
| 41 | TFT_RST | Out | ILI9341/ST7789V RST | 10kΩ pull-up to 3.3V |
| 42 | TOUCH_CS | Out | XPT2046 T_CS | GPIO out |
| 47 | RPi_TX | Out | RPi GPIO15 (RX) | UART1 TX, 3.3V, 921600 baud |
| 48 | RPi_RX | In | RPi GPIO14 (TX) | UART1 RX, 3.3V |

# 5. UAS signal chain

1MHz acoustic signal chain for particle attenuation measurement. All on 3.3V rail. AD9833 can sweep 0.5–3MHz in firmware for spectral data — same hardware. All gains verified against 3.3V supply rail limits.

## 5.1 AD9833BRMZ — signal generator

| Parameter | Requirement |
| --- | --- |
| IC | AD9833BRMZ (MSOP-10) |
| Supply | 3.3V — both VDD pins |
| SPI | GPIO38 (FSYNC), GPIO36 (SCLK), GPIO35 (SDATA) — write-only. SPI Mode 2 (CPOL=1, CPHA=0). |
| RSET | 6.8kΩ 1% 0402 to AGND → IOUT ≈ 3.02mA → 604mVpp across 200Ω load |
| COMP | 10nF X7R 0402 to AGND |
| Output coupling | 100nF X7R 0402 from IOUT load to Tx amp input |
| Decoupling | 100nF X7R per VDD pin + 10µF X5R 0805 bulk |

## 5.2 Tx amplifier — IC1 channel A, G = 4.9

| Parameter | Requirement |
| --- | --- |
| IC | OPA2354 SOIC-8, channel A of IC1. 250MHz GBW. 3.3V single supply. |
| Bias | Two 10kΩ (3.3V → non-inv → GND) — 1.65V mid-rail |
| Input coupling | 100nF X7R from AD9833 |
| Gain | Rf = 390Ω 1%, Rg = 100Ω 1%. G = 4.9. Output = 2.96Vpp. Peak = 3.13V < 3.3V. ✓ |
| Output | 47Ω series + 100nF coupling to J1 SMA |

## 5.3 IC1 channel B — unused, must be tied off

- Pin 5 (non-inv input ch-B): connect to 1.65V mid-rail node from existing 10kΩ/10kΩ divider

- Pin 6 (inv input ch-B): connect directly to pin 7 (output ch-B) — unity-gain follower. Prevents unused channel oscillating.

## 5.4 Rx input protection — J2 SMA → R_in → D1 BAV99

| Component | Value | Function |
| --- | --- | --- |
| J2 | SMA edge-launch | Rx transducer input |
| R_in | 100Ω 0402 | Series protection — limits fault current |
| D1 | BAV99 SOT-23 | Dual diode clamp to 0V and 3.3V |

## 5.5 Rx two-stage amplification — IC2, total G = 100 (+40 dB)

| Stage | IC | Rf | Rg | Gain | Notes |
| --- | --- | --- | --- | --- | --- |
| Rx stage 1 | IC2 channel A | 910Ω 1% | 100Ω 1% | G = 10.1 | BW = 25MHz >> 1MHz ✓ |
| Rx stage 2 | IC2 channel B | 910Ω 1% | 100Ω 1% | G = 10.1 | Total: 100× — max non-clipping Rx input = 32mVpp |

Both stages have 10kΩ/10kΩ mid-rail bias dividers and 100nF AC coupling at input and between stages. IC2 V+ decoupling: 100nF + 10µF bulk. If received signal is too weak in lab, replace Rf 910Ω with 3.9kΩ in IC2 ch-A only for G=40 — single resistor swap, no PCB change.

## 5.6 Envelope detector and ADC input

| Component | Value | Function |
| --- | --- | --- |
| D2 | BAT54 SOD-123 | Envelope rectifier, Vf ≈ 0.2V. Anode: IC2 ch-B output. Cathode: RC filter. |
| R_env | 10kΩ 0402 | RC discharge: τ = 100µs, fc = 1.6kHz, ripple at 1MHz = 0.16% |
| C_env | 10nF X7R 0402 | RC filter in parallel with R_env. Settling time (5τ) = 500µs. |
| D3 | BAV99 SOT-23 | Hard clamp at GPIO1 — overvoltage protection |
| C_adc | 100nF X7R 0402 | GPIO1 to AGND noise filter |

No voltage divider at GPIO1. Max envelope ≈ 1.4V. ESP32 ADC in 11dB attenuation mode accepts 0–3.1V — 1.4V fits with 2.2× headroom. ADC resolution: 0.757mV/LSB. Firmware: configure ADC1_CH0 in ADC_ATTEN_DB_11.

# 6. Motor drivers — 2× TMC2209-LA

Two TMC2209-LA QFN-28 ICs drive two NEMA17 stepper motors. Shared half-duplex UART on GPIO4. StallGuard4 provides motor-load viscosity proxy via SG_RESULT in DRVSTATUS register. Hot plate reflow with solder paste and stencil resolves the QFN hand-soldering concern.

## 6.1 UART addressing

| Driver | Ref | MS1 | MS2 | UART address |
| --- | --- | --- | --- | --- |
| Motor 1 | U5 | GND (tie direct) | GND (tie direct) | 0 |
| Motor 2 | U6 | 3.3V via 10kΩ | GND (tie direct) | 1 |

1kΩ series resistor between GPIO4 and UART junction. Both PDN_UART pins connect to junction. UART1 TX and RX both mapped to GPIO4 via GPIO matrix in firmware. Enable SpreadCycle via UART GCONF register at startup — StallGuard inactive in default StealthChop mode.

## 6.2 Power and critical pins — each TMC2209

| Pin | Connection | Notes |
| --- | --- | --- |
| VM | 24V motor rail | 100µF electrolytic + 100nF X7R at each IC VM pin — separate, not shared |
| VCC_IO | 3.3V | Logic supply |
| VCP | 100nF X7R 0402 to GND | CRITICAL — charge pump for high-side gate driver. Without this cap the motor will not run. Within 2mm. |
| SPREAD | 3.3V via 10kΩ | Forces SpreadCycle in hardware — StallGuard requires SpreadCycle. Failsafe if firmware fails to configure GCONF. |
| EN | 10kΩ pull-up to 3.3V at IC | Active LOW. Pull-up overrides GPIO7/10 power-up glitch. |
| CLK | Floating (DNP pad to GND) | Enables internal 12MHz oscillator |
| VREF | Floating (DNP test point) | Self-biases to VCC_IO/2 = 1.65V. Current via UART IRUN/IHOLD. |
| Exposed pad | GND pour via ≥4 vias | Thermal path — mandatory connection to GND |

## 6.3 Control signals — U5 (Motor 1)

| Pin | GPIO | Notes |
| --- | --- | --- |
| STEP | GPIO5 | LEDC PWM |
| DIR | GPIO6 | GPIO out |
| EN | GPIO7 | 10kΩ pull-up to 3.3V at IC |
| PDN_UART | GPIO4 via 1kΩ | Address 0 — MS1=GND, MS2=GND |

## 6.4 Control signals — U6 (Motor 2)

| Pin | GPIO | Notes |
| --- | --- | --- |
| STEP | GPIO8 | LEDC PWM |
| DIR | GPIO9 | GPIO out |
| EN | GPIO10 | 10kΩ pull-up to 3.3V at IC |
| PDN_UART | GPIO4 via 1kΩ | Address 1 — MS1=3.3V via 10kΩ, MS2=GND |

## 6.5 Motor connectors — JST-XH 6-pin (changed from v2.1)

Each motor now uses one 6-pin JST-XH connector instead of two 2-pin connectors. Pins 1–4 carry the four motor wires. Pins 5–6 are no-connect (NC) — reserved for future use or 6-wire motor center tap.

| Ref | Label | Type | Pin assignment |
| --- | --- | --- | --- |
| J4 | Motor 1 | JST-XH 2.5mm 6-pin | Pin 1: OA1 → TMC2209 U5. Pin 2: OA2 → U5. Pin 3: OB1 → U5. Pin 4: OB2 → U5. Pin 5: NC. Pin 6: NC. |
| J5 | Motor 2 | JST-XH 2.5mm 6-pin | Pin 1: OA1 → TMC2209 U6. Pin 2: OA2 → U6. Pin 3: OB1 → U6. Pin 4: OB2 → U6. Pin 5: NC. Pin 6: NC. |

Current sense resistors: RS1 = RS2 = 0.11Ω 1% (2512) on U5 coil sense pins. RS3 = RS4 = 0.11Ω 1% on U6. Kelvin connections required. Motor trace width ≥0.5mm for all 8 coil traces. Mark motor connector polarity and pin 1 position on silkscreen.

# 7. Limit switches

Two NO microswitches detect end-of-travel. Switch closure pulls GPIO to GND. Firmware immediately kills STEP PWM on trigger.

| GPIO | Ref | Connector | Wiring |
| --- | --- | --- | --- |
| GPIO14 | LIMIT_M1 | J6 — JST-XH 2.5mm 2-pin | Pin 1: GND. Pin 2: GPIO14. 100nF from GPIO14 to GND. |
| GPIO15 | LIMIT_M2 | J7 — JST-XH 2.5mm 2-pin | Pin 1: GND. Pin 2: GPIO15. 100nF from GPIO15 to GND. |

Note: connector references updated from v2.1 (were J8/J9, now J6/J7) due to motor connector consolidation.

# 8. TFT touch-screen interface

Shopee module: SPI TFT Serial Port Module 5V/3.3V PCB Adapter with Micro SD slot — ILI9341 or ST7789V display + XPT2046 touch. Power from 3.3V pin directly on module.

## 8.1 Module connection (10-pin 2.54mm socket — J9)

| Pin | Signal | GPIO | Notes |
| --- | --- | --- | --- |
| 1 | VCC | 3.3V | Connect to 3.3V rail pin on module — bypasses onboard LDO |
| 2 | GND | GND |  |
| 3 | CS (display) | GPIO39 |  |
| 4 | RST | GPIO41 | 10kΩ pull-up to 3.3V |
| 5 | DC | GPIO40 |  |
| 6 | MOSI | GPIO35 | SPI2 shared |
| 7 | SCK | GPIO36 | 40MHz display, 2MHz touch — firmware must switch |
| 8 | LED (backlight) | 3.3V via 47Ω | Always-on |
| 9 | T_CS | GPIO42 | Touch chip select |
| 10 | T_IRQ | GPIO21 | 100nF to GND at GPIO21 |

SD card CS pin: pull to 3.3V via 10kΩ if exposed on module. Floating SD CS interferes with SPI bus. Confirm ILI9341 vs ST7789V on received module before writing display init code — initialization sequences differ.

## 8.2 Physical buttons

| Ref | Label | GPIO | Connector |
| --- | --- | --- | --- |
| SW1 | UP | GPIO16 | J10 — 3-pin 2.54mm |
| SW2 | DOWN | GPIO17 | J11 — 3-pin 2.54mm |
| SW3 | SELECT | GPIO18 | J12 — 3-pin 2.54mm |

# 9. External connectors — complete list (v2.2 numbering)

| Ref | Label | Type | Connected to |
| --- | --- | --- | --- |
| J1 | TX Transducer | SMA edge-launch | IC1 ch-A output (after 47Ω + 100nF) |
| J2 | RX Transducer | SMA edge-launch | R_in 100Ω → D1 BAV99 |
| J3 | RPi UART | JST-SH 1.0mm 4-pin | GPIO47 (TX), GPIO48 (RX), GND — no power to RPi |
| J4 | Motor 1 | JST-XH 2.5mm 6-pin | Pins 1–4: TMC2209 U5 OA1/OA2/OB1/OB2. Pins 5–6: NC. |
| J5 | Motor 2 | JST-XH 2.5mm 6-pin | Pins 1–4: TMC2209 U6 OA1/OA2/OB1/OB2. Pins 5–6: NC. |
| J6 | Limit switch M1 | JST-XH 2.5mm 2-pin | GPIO14 + GND |
| J7 | Limit switch M2 | JST-XH 2.5mm 2-pin | GPIO15 + GND |
| J8 | LED panel | JST-XH 2.5mm 2-pin | 5V + GND — always-on |
| J9 | TFT module | 2.54mm 10-pin socket | SPI2 + control — section 8.1 |
| J10 | Button UP | 2.54mm 3-pin header | GPIO16 |
| J11 | Button DOWN | 2.54mm 3-pin header | GPIO17 |
| J12 | Button SELECT | 2.54mm 3-pin header | GPIO18 |
| J13 | USB-C programming | USB-C receptacle | GPIO19 D−, GPIO20 D+, VBUS via D6 Schottky |
| J14 | 24V PSU input | 3.5mm 2-pin screw terminal | Via Q1 AO4407A, F1 polyfuse, D5 TVS |

# 10. PCB layout rules

## 10.1 Layer stack

- 2-layer FR4, 1oz copper, ENIG. Top: all SMD + signal routing. Bottom: ground plane + power routing.

- UAS analog signals on top layer only — no vias in UAS signal paths.

## 10.2 Antenna keepout

- No copper, traces, vias, or components within 15mm of ESP32 PCB antenna on ALL layers.

- Place module at board edge with antenna pointing outward.

## 10.3 Power section

- Q1 AO4407A: D7 Zener (BZX84C15) cathode to Source side, anode to Gate. R_gate 10kΩ from Gate to GND. Place D7 within 3mm of Q1.

- SS34A catch diode within 5mm of LM2596 SW pin. 10Ω + 1nF snubber from SW pin to GND.

- AMS1117 SOT-223 tab: ≥100mm² top copper + ≥4 thermal vias to bottom ground plane.

## 10.4 UAS signal chain

- IC1 (Tx) and IC2 (Rx): ≥15mm physical separation.

- J2 → R_in → D1 → IC2 ch-A input: under 20mm, no vias, ground guard traces.

- Envelope RC within 5mm of IC2 ch-B output. GPIO1 trace under 10mm, no vias.

## 10.5 TMC2209 motor drivers

- QFN-28: use solder paste + stencil + hot plate reflow. Verify GND continuity on thermal pad after reflow.

- VCP cap (100nF) within 2mm of VCP pin of each IC — critical, separate per IC.

- VM decoupling (100µF + 100nF) directly at each IC VM pin — separate per IC.

- Motor coil traces (J4/J5 outputs): ≥0.5mm width. Current sense resistors: Kelvin connections.

- Motor connectors J4 and J5: mark pin 1 position on silkscreen. NC pads on pins 5–6 should be connected to GND or left as no-connect — do not leave floating copper.

# 11. Bill of materials summary (v2.2)

| Qty | Ref | Description | Part | Package |
| --- | --- | --- | --- | --- |
| 1 | U1 | ESP32-S3 MCU module | ESP32-S3-WROOM-1-N8 | Castellated 18×25.5mm |
| 1 | Q1 | P-MOSFET reverse polarity guard | AO4407A | SOIC-8 |
| 1 | D7 | Gate Zener clamp (NEW) | BZX84C15 15V | SOT-23 |
| 1 | U2 | Buck 24V→5V | LM2596-5.0 | TO-263-5 |
| 1 | U3 | LDO 5V→3.3V | AMS1117-3.3 | SOT-223 |
| 1 | U4 | DDS signal generator | AD9833BRMZ | MSOP-10 |
| 2 | IC1, IC2 | Dual op-amp (UAS chain) | OPA2354 | SOIC-8 |
| 2 | U5, U6 | Stepper motor driver | TMC2209-LA | QFN-28 — reflow only |
| 2 | D1, D3 | Dual diode clamp | BAV99 | SOT-23 |
| 1 | D2 | Envelope rectifier | BAT54 | SOD-123 |
| 1 | D4 | Buck catch diode | SS34A (replaces 1N5822) | DO-214AB (SMA) |
| 1 | D5 | TVS input protection | SMBJ28A | DO-214AA (SMA) |
| 1 | D6 | USB VBUS Schottky | 1N5819 | SOD-123 |
| 1 | F1 | Input polyfuse | 1A hold | 1812 |
| 1 | L1 | Buck inductor | 100µH ≥3A saturation | SMD (e.g. Bourns SRR1260) |
| 2 | J4, J5 | Motor connectors (6-pin, CHANGED) | JST-XH 2.5mm 6-pin | Vertical, 4 pins used |
| 4 | RS1–RS4 | Current sense resistors | 0.11Ω 1% | 2512 |
| 1 | J13 | USB-C connector | USB Type-C receptacle | SMD |
| 2 | SW_boot, SW_reset | Tactile switches | 6×6mm | SMD |
| 2 | J1, J2 | SMA connectors | Edge-launch SMA | PCB edge |
| Note | — | v2.1 AO3401A SOT-23 REMOVED — replaced by AO4407A SOIC-8 + BZX84C15 Zener | — | — |
| Note | — | v2.1 J4/J5/J6/J7 four 2-pin connectors REMOVED — replaced by J4/J5 two 6-pin connectors | — | — |

# 12. Pre-submission checklist (v2.2)

## 12.1 Reverse polarity circuit (AO4407A — updated from v2.1)

- Q1 AO4407A: Source pins (2–4) to J14 V+, Drain pins (5–8) to protected rail, Gate pin (1) to R_gate/D7 junction

- R_gate 10kΩ from Gate to GND — confirmed fitted

- D7 BZX84C15: cathode to Source (J14 V+ line), anode to Gate. Without D7, Vgs = −24V exceeds ±20V AO4407A limit.

- D7 placed within 3mm of Q1

## 12.2 Motor connectors (6-pin — changed from v2.1)

- J4 (Motor 1): JST-XH 6-pin. Pins 1–4 connected to U5 OA1/OA2/OB1/OB2. Pins 5–6 NC.

- J5 (Motor 2): JST-XH 6-pin. Pins 1–4 connected to U6 OA1/OA2/OB1/OB2. Pins 5–6 NC.

- Pin 1 position marked on silkscreen for both J4 and J5

- Motor traces ≥0.5mm width from J4/J5 to TMC2209 motor output pins

- Old J4/J5/J6/J7 (four 2-pin connectors) are removed from design — confirm not present in schematic or layout

## 12.3 TMC2209 reflow (QFN-28)

- Hot plate reflow with solder paste + stencil confirmed as assembly method

- VCP 100nF per IC — within 2mm. VM decoupling 100µF + 100nF per IC.

- Thermal pad: test GND continuity on IC body after reflow

- SPREAD pin: 10kΩ pull-up to 3.3V. EN pull-up 10kΩ at each IC.

- UART junction: single 1kΩ from GPIO4, both PDN_UART to junction.

## 12.4 UAS chain

- GPIO1 = ADC1_CH0. No voltage divider. IC1 ch-B tied off. Rf values: IC1-A 390Ω, IC2-A/B 910Ω.

- D7 in this checklist refers to gate Zener. Envelope D2 = BAT54. ADC clamp D3 = BAV99.

## 12.5 Connector numbering (v2.2)

- J1/J2: SMA (Tx/Rx). J3: RPi UART JST-SH. J4: Motor 1 6-pin. J5: Motor 2 6-pin.

- J6: Limit M1. J7: Limit M2. J8: LED panel. J9: TFT 10-pin socket.

- J10/J11/J12: Button headers. J13: USB-C. J14: PSU screw terminal.

# 13. KiCad footprint reference

All footprint strings in the format Library:FootprintName as entered in KiCad symbol footprint field.

## 13.1 ICs and active components

| Ref | Part | KiCad footprint string |
| --- | --- | --- |
| U1 | ESP32-S3-WROOM-1-N8 | RF_Module:ESP32-S3-WROOM-1 |
| U2 | LM2596-5.0 | Package_TO_SOT_SMD:TO-263-5_TabPin3 |
| U3 | AMS1117-3.3 | Package_TO_SOT_SMD:SOT-223-3_TabPin2 |
| U4 | AD9833BRMZ | Package_SO:MSOP-10_3x3mm_P0.5mm |
| IC1, IC2 | OPA2354 (SOIC-8) | Package_SO:SOIC-8_3.9x4.9mm_P1.27mm |
| U5, U6 | TMC2209-LA (QFN-28) | Package_DFN_QFN:QFN-28-1EP_5x5mm_P0.5mm_EP3.7x3.7mm_ThermalVias |
| Q1 | AO4407A (SOIC-8 P-MOSFET) | Package_SO:SOIC-8_3.9x4.9mm_P1.27mm |

## 13.2 Diodes and protection

| Ref | Part | KiCad footprint string |
| --- | --- | --- |
| D1, D3 | BAV99 dual diode clamp | Diode_SMD:D_SOT-23 |
| D2 | BAT54 Schottky (envelope) | Diode_SMD:D_SOD-123 |
| D4 | SS34A Schottky (buck catch diode) | Diode_SMD:D_SMA |
| D5 | SMBJ28A TVS | Diode_SMD:D_SMA |
| D6 | 1N5819 Schottky (USB VBUS) | Diode_SMD:D_SOD-123 |
| D7 | BZX84C15 15V Zener (gate clamp) | Diode_SMD:D_SOT-23 |
| F1 | Polyfuse 1A (1812) | Fuse:Fuse_1812_4532Metric |

## 13.3 Inductors

| Ref | Part | KiCad footprint string |
| --- | --- | --- |
| L1 | 100µH ≥3A SMD inductor (e.g. Bourns SRR1260-101Y) | Inductor_SMD:L_Bourns-SRR1260 |

Verify footprint against chosen inductor datasheet. Bourns SRR1260 footprint: 12.7×12.7mm body. If using alternative inductor, check pad layout against manufacturer datasheet and create custom footprint if needed.

## 13.4 Resistors

| Ref | Value | KiCad footprint string |
| --- | --- | --- |
| All standard 0402 resistors (10kΩ, 5.1kΩ, 4.7kΩ, 1kΩ, 330Ω, 200Ω, 100Ω, 47Ω, 10Ω) | Various | Resistor_SMD:R_0402_1005Metric |
| 1% precision 0402 (390Ω, 910Ω ×2, 100Ω ×3, 6.8kΩ) | Precision values | Resistor_SMD:R_0402_1005Metric |
| RS1–RS4 current sense | 0.11Ω 1% | Resistor_SMD:R_2512_6332Metric |

## 13.5 Capacitors — ceramic SMD

| Value / type | KiCad footprint string | Notes |
| --- | --- | --- |
| 100nF X7R 0402 | Capacitor_SMD:C_0402_1005Metric | 26 units total — decoupling, debounce, coupling |
| 10nF X7R 0402 | Capacitor_SMD:C_0402_1005Metric | 2 units — C_env (envelope) + C_COMP (AD9833) |
| 1nF X7R 0402 | Capacitor_SMD:C_0402_1005Metric | 1 unit — SW snubber (with 10Ω) |
| 1µF X7R 0402 | Capacitor_SMD:C_0402_1005Metric | 1 unit — C_en (ESP32 EN RC) |
| 10µF X5R 0805 | Capacitor_SMD:C_0805_2012Metric | 5 units — bulk bypass for ESP32 (×2), AD9833, IC1, IC2 |

## 13.6 Capacitors — tantalum and electrolytic

| Value / type | KiCad footprint string | Notes |
| --- | --- | --- |
| 10µF 10V tantalum (Case A) | Capacitor_Tantalum_SMD:CP_EIA-3216-18_Kemet-A | 2 units — AMS1117 input and output stability caps |
| 100µF 50V electrolytic (radial) | Capacitor_THT:C_Radial_D8.0mm_P3.5mm | 1 unit — LM2596 input bulk |
| 220µF 10V low-ESR electrolytic (radial) | Capacitor_THT:C_Radial_D8.0mm_P3.5mm | 1 unit — LM2596 output bulk |
| 100µF 35V electrolytic (radial) | Capacitor_THT:C_Radial_D6.3mm_P2.5mm | 2 units — TMC2209 VM bulk (one per IC) |

Radial through-hole electrolytic caps are hand-soldered after hot plate reflow. Place on board such that they can be inserted and soldered from the top side.

## 13.7 Connectors

| Ref | Description | KiCad footprint string |
| --- | --- | --- |
| J1, J2 | SMA PCB edge-launch (Tx / Rx transducer) | Connector_Coaxial:SMA_Molex_0732511150_EdgeMount |
| J3 | JST-SH 1.0mm 4-pin (RPi UART) | Connector_JST:JST_SH_BM04B-SRSS-TB_1x04-1MP_P1.00mm_Vertical |
| J4, J5 | JST-XH 2.5mm 6-pin (Motor 1 / Motor 2) | Connector_JST:JST_XH_B6B-XH-A_1x06_P2.50mm_Vertical |
| J6, J7 | JST-XH 2.5mm 2-pin (Limit switches M1 / M2) | Connector_JST:JST_XH_B2B-XH-A_1x02_P2.50mm_Vertical |
| J8 | JST-XH 2.5mm 2-pin (LED panel) | Connector_JST:JST_XH_B2B-XH-A_1x02_P2.50mm_Vertical |
| J9 | 2.54mm 10-pin female socket (TFT module) | Connector_PinHeader_2.54mm:PinSocket_1x10_P2.54mm_Vertical |
| J10, J11, J12 | 2.54mm 3-pin male header (button headers ×3) | Connector_PinHeader_2.54mm:PinHeader_1x03_P2.54mm_Vertical |
| J13 | USB-C receptacle (programming) | Connector_USB:USB_C_Receptacle_HRO_TYPE-C-31-M-12 |
| J14 | 3.5mm 2-pin screw terminal (PSU input) | TerminalBlock_Phoenix:TerminalBlock_Phoenix_PT-1,5-2-3.5_1x02_P3.50mm_Horizontal |

## 13.8 Switches and LEDs

| Ref | Description | KiCad footprint string |
| --- | --- | --- |
| SW_boot, SW_reset | 6×6mm SMD tactile switch | Button_Switch_SMD:SW_SPST_EVQP2_6x6mm |
| LED_pwr | Red LED 0805 | LED_SMD:LED_0805_2012Metric |
| LED_stat | Green LED 0805 | LED_SMD:LED_0805_2012Metric |

— End of document —